|  | SOP 4.09 Explosive, Reactive & Ammunition Wastes General Guidance for HHW Programs |
| --- | --- |

Note: This guidance document is for informational purposes only and outlines basic employer requirements. The MPCA makes no guarantee that this document satisfies the requirements of its users ensuring compliance. It is the sole responsibility of your specific Household Hazardous Waste (HHW) Program employer(s) to determine if OSHA, DOT and RCRA requirements have been met.

# 1.	Introduction

This Standard Operating Procedure (SOP) provides guidance to ensure proper acceptance, processing and shipment of unstable and ammunition waste types, supporting safe and efficient facility operations. Explosives or chemically reactive materials are not typically accepted at this HHW Facility due to the potential for violent reactions. However, such wastes may occasionally be presented for disposal. For this SOP, all potentially explosive, reactive, or incompatible chemicals or wastes will be referred to as ‘unstable’. Facility staff must follow established safe work practices to minimize the risk of injury while managing unstable waste materials.

In the event of a suspected unstable waste incident, staff shall immediately notify law enforcement and the State Duty Officer (800.422.0798). Emergency Response and/or Bomb Squad staff may be dispatched to respond.

# 2.	Training requirements

This Facility shall maintain a written training program and/or SOP that outlines the proper management of unstable wastes and must include function specific training covering safety protocols, regulatory compliance, and emergency response procedures.

2.1	Designation of Responsible Staff. The [Facility Manager] the designated HHW Program staff responsible to ensure:

- All facility staff complete required training

- Response protocols and policies related to staff conduct are enforced.

- Professional judgment is used, in coordination with experienced personnel to assess and respond appropriately to unique or potential emergency situations.

2.2	Training requirements. All staff must receive training specific to the handling and response needed to manage unstable waste materials, including:

- Complete training within six months of hire.

- Untrained staff work under direct supervision of trained staff.

- Staff must demonstrate a thorough understanding of job-specific tasks, program requirements and emergency response expectations.

# 3.   Safety requirements

Facility staff must remain vigilant for immediate hazards, suspicious behavior, and potentially unstable waste. In such cases, the following procedures apply:

3.1	Staff handling HHW must wear appropriate PPE, including nitrile gloves, Tyvek or chemically resistant apron/sleeves, and eye protection.

3.2 	Minimize any movement of suspected unstable wastes  to prevent triggering reactions.

3.3 	Only trained emergency response personnel or hazardous waste contractors may handle incidents beyond staff capabilities. Agreements with emergency responders must be established and outlined in the Facility’s Emergency Contingency Plan.

3.4 	Designate an outdoor area to temporarity store incoming potentially unstable or explosive materials. Ensure the area is:

- located at least 50 feet from the Facility or meet local fire/building code requirements.

- designed to withstand blast pressure to protect people and property.

- constructed using an empty flammable storage unit or a pit (e.g., steel drum or cement culvert) buried in sand with one foot of sand or inert absorbent at the base.

- clearly marked: “Secure Area – Authorized Personnel Only.”

# 4.	Responding to an unstable waste incident

4.1 	Upon drop-off, staff must immediately assess any waste suspected to be unstable. Efforts should be made to gather as much information as possible from the participant regarding the nature and origin of the waste. THE FONT SIZE SEEMS TO CHANGE IN THIS AREA

4.2 	Inspect the container for structural integrity and any identifying labels or markings. Use this information to determine whether the waste requires segregation from other materials or if the situation constitutes an emergency. Refer to Attachment A for a list of suspicious items and chemicals.

4.3 	Any container exhibiting crystal formation on the cap or inside the container must be treated as potentially shock sensitive and receive immediate attention; follow all procedures outlined in Section 6 prior to handling or relocating the unstable waste.

4.4 	If adequate information about the waste cannot be obtained, the material must be treated as a potential hazard. Immediately isolate the waste in a designated, secured processing area. Prompt and appropriate action is critical upon identification of potentially unstable materials.

# 5.	Common Unstable Waste Types

5.1 	Explosive Wastes -Staff must not disturb any container suspected of containing explosive materials, as even minimal physical movement may result in detonation. Refer to Attachment A or consult the State’s Hazardous Waste Disposal Contractor's Hazardous Categorization Training Manual for guidance. If a container exhibits any markings listed in Attachment A, immediately clear and secure the Facility area and notify emergency authorities. Use professional judgement to determine the best response.

5.2 	Reactive Wastes - Staff shall remain vigilant for signs of chemical reactions in waste materials. A visual inspection of each container should be performed to detect signs of active reactions, such as:

- Smoking or vapor emission

- Fizzing or bubbling

5.3 	Shock-Sensitive Wastes - Certain chemicals can explode when exposed to shock, friction, or vibration; see Attachment C.

- Do not move or handle any waste suspected to be shock-sensitive unless you are fully certain it is safe to do so; see Attachment D.

- Do not assume a chemical is safe just because it arrived at the Facility without incident.

- Handle such materials minimally and with extreme caution until appropriate hazardous materials personnel arrive.

5.4 	Ammunition and Explosive Devices -Unusable ammunition, explosive devices (e.g., flash-bang or      stinger grenades), fireworks, and incendiaries are considered hazardous wastes unless formally evaluated and deemed non-hazardous. These items must only be transported off-site by the State’s approved hazardous waste disposal contractor. In some cases, viable ammunition may be accepted for training purposes by local law enforcement agencies or gun clubs. If the Minnesota Bomb Squad is already on-site for other operations, they may be able to accept waste ammunition for proper disposal.

5.5 	Fireworks and Flares -Some firework materials and marine flares may be wetted to minimize explosive risk prior to handling or disposal. For proper procedures, refer to or contact the State’s Hazardous Waste Disposal Contractor’s Hazardous Categorization Training Manual.

5.6 	Unknown or Abandoned Wastes -Unlabeled, unknown, or abandoned materials may be accepted by the Program and may be commingled with otherHHW, provided they present no immediate threat.

5.7  	Stable Materials -If a material is confirmed to be chemically stable, it may be safely processed and/or transferred to the appropriate Facility storage area.

# 6.	Emergency response

Upon identification and confirmation of an unstable waste, staff must immediately initiate the following emergency response:

  - Emergency Notification - Contact local emergency responders and the Minnesota State Duty Officer (1-800-422-0798).

- Clearly describe the situation, including the nature and location of the unstable wastes and follow all instructions.

- If warranted, emergency responders will notify the MN Bomb Squad to respond to the hazard; only trained personnel are permitted to handle, remove and detonate explosive materials in a controlled environment.

  - Evacuation Procedures - If the unstable container is deemed too hazardous to move:

- Evacuate the immediate area and provide a safety radius of 300 feet away from the unstable container.

- Restrict access to the area and Inform all nearby persons of the dangerous situation.

- Staff may choose to continue accepting waste from the public during this time using an alternate receiving location (e.g., parking lot, driveway, transfer station). It may be necessary to temporarily suspend Facility operations until the incident is resolved.

Attachment A

Waste items demanding immediate staff response

Staff shall be constantly alert for unstable wastes labeled with the following chemicals:

1.	Anhydrous ether

an old can, labeled “ether” may be explosive

staff shall not move or open these containers, especially if there is no participant history

unstable crystals can form around inside container threads, in lids, or in solution which may not be visible

2.	Blasting caps: fuse type or unknown type

3.	Bombs

metal or PVC pipe bombs

may be in any shape

may be wrapped in duct tape

gel tubes or slurries (pink or white tubes sausage-looking: generally, >2 inches in diameter)

homemade explosive devices like tennis balls wrapped in duct tape

4.	Dynamite

brown or tan casing (typically has a 5-star fold at one end)

considered extremely dangerous if wet, glistening, or soft to the touch

may be labeled with unusual or hand-written markings indicating explosive hazards

key words: nitro, explosive, shock sensitive, material should only be handled by a trained technician

evidence that dynamite may be present includes detonation cord, electric blasting caps, shock tube blasting caps

5.	Flares

have strikers that will ignite through friction

have orange or red casings (dynamite is usually brown)

must be separated from other materials

flares may not need to be managed as explosive waste types

6.	Flash powder or photo-flash

7.	Fireworks

smokeless powder, pryrodex (as indicated on original container label)

ammonium nitrate (hobby powder and used as a fertilizer)

nitromethane (hobby rocket pellets); liquid form/RC Fuel is not explosive

boosters (typically have no markings)

ammunition or fireworks may not need to be managed as an explosive waste type

8.	Military ordnance: grenades or shells

9.	Nitrogen: container shows physical evidence it’s been altered

10.	Picric acid

unknown, dry or white powders

extremely shock sensitive materials

commonly found in medical or research laboratories

used as a dye, chemical reagent, or in explosives/firework manufacturing

in dry form, may explode if exposed to minute amounts of heat, flame, or shock

is a strong oxidant which may react violently if exposed to metals, metal salts, amines, ammonia, bases, plaster, or concrete

key words: Trinitrophenol, Flammable, Dangerous When Wet, Shock Sensitive, Keep Dry

11.	Sodium amide

if uncontaminated, color will be a white to gray crystalline material

has an ammonia odor

the pure material is water reactive

if old or degraded, color will be yellow to brown with discoloration beginning at the bottom and is a potential fire and/or explosion hazard

12.	Unrecognizable reactive wastes

inside lab or industrial containers

covered in wax

sealed in metal or fiber containers

often less than a quart in size

markings indicating the container (or need to be refrigerated)

13. Sodium Azide

Airbags and seatbelt pretensioners

Attachment B – Unstable Chemical Waste List

| Unstable chemical list is not all-inclusive, but includes wastes commonly processed at HHW facilities. Please use this list as a general reference only. |  |
| --- | --- |
| Acetal | Hydrogen peroxide (>8%) |
| Activated charcoal | Isopropyl ether |
| Aldehydes | Lithium metal (powder) |
| Alkenes | Metal peroxides |
| Ammonium nitrate | Metal turnings (non-DWW) |
| Benzoyl peroxide pastes | Methyl acetylene |
| Benzylic hydrogen atoms | Methyl ethyl ketone peroxide |
| Calcium hypochlorite (solid) | Naphthalene (mothballs/flakes) |
| Calcium metal (powder) | Nitrates |
| Charcoal (briquettes/screen/shell) | Paraformaldehyde powder (found in 2-part resin glue) |
| Chromates | Perchlorates |
| Cumene | Permanganates |
| Cyclohexene | Phosphorus black, red, white |
| Cyclooctene | Picric acid (wet) |
| Decahydronaphthalene | Pool chemicals (solid) |
| Decalin | Potassium dichromate |
| Diacetylene | Potassium metal |
| Dicyclopentadiene | Potassium nitrate |
| Diethyl ether | Sodium amide |
| Diethylene glycol | Sodium hydrosulfite (iron out / water softener cleaner) |
| Diisopropyl ether | Sodium hypochlorate |
| Dimethyl ether | Sodium hypochlorite (solid) |
| Dinitro compounds (wet) | Sodium nitrate |
| Dinitro compounds (wet) | Sodium perchlorate |
| Dioxane | Sodium sulfide (<30% water of crystallization) |
| Divinyl acetylene | Tetrahydrofuran (THF) |
| Ethers, cyclic ethers, containing primary & secondary alcohol groups | Tetrahydronaphthalene |
| Ethylene glycol dimethyl ether (glyme) | Tetralin |
| Fiberglass ardeners/resins | Vinyl, vinyl acetate & vinylidene compounds |
| Fuses | Vinylidene chloride |

Attachment C

Reactive Chemical Guidance

| Keep these… + | Away from these… | Or you may get these… |
| --- | --- | --- |
| Acids | Bases | Heat violent reaction |
| Acids or bases | Reactive metals (aluminum, beryllium, calcium, lithium, potassium, magnesium, sodium, zinc powder), metal hydrides | Fire Explosion Hydrogen gas |
| Water or alcohols | Concentrated acids or bases (calcium, lithium, potassium, metal hydrides, other waste reactives) | Heat Fire Explosion Flammable and toxic gases |
| Reactive organic compounds or solvents (alcohols, aldehydes, nitrated hydrocarbons) | Concentrated acids or bases, reactive metals and metal hydrides | Fire Explosion |
| Cyanide or sulfide solutions | Acids | Toxic hydrogen Cyanide Sulfide gas |
| Strong oxidizers (chlorates, chlorine, chlorites, chromic acid, hypochlorites, nitrates, perchlorates, permanganates, peroxides) | Organic acids, concentrated mineral acids, reactive metals, metal hydrides, reactive organic compounds or solvents, flammable or combustible waste | Fire Explosion |

Attachment D

Shock Sensitive Chemical Guidance List

Acetylides

Aluminum ophorite explosive

Amatol

Ammonal

Ammonium nitrate

Ammonium perchlorate

Ammonium picrate

Ammonium salt lattice

Butyl tetryl

Calcium nitrate

Copper acetylide

Cyanuric triazide

Cyclotrimethylenetrinitramine

dinitroethyleneurea

dinitroglycerine

dinitrophenol

dinitrophenolates

dinitrophenyl hydrazine

dinitrotoluene

dipicryl sulfone

dipicrylamine

erythritol tetranitrate

fulminate of mercury

fulminate of silver

fulminating gold

fulminating mercury

fulminating platinum

gelatinized nitrocellulose

guanyl nitrosamino guanyltetrazene

guanyl nitrosamino guanylidene hydrazine

guanylidene

heavy metal azides

hexanite

hexanitrodiphenylamine

hexanitrostilbene

hexogen

hydrazine mixtures

hydrazinium nitrate

lead azide

lean mannite

lead mononitroresorcinate

lead picrate

lead salts

lead styphnate

magnesium ophorite

mannitol hexanitrate

mercury oxalatemercury tartrate

nitrated carbohydrate

nitrated glucoside

nitrated polyhydric alcohol

nitrogen trichloride

nitrogen tri-iodide

nitroglycerin

nitroglycide

nitroglycol

nitroguanidine

nitroparaffins

nitronium perchlorate

nitrotoluene

nitrourea

organic amine nitrates

organic nitramines

organic peroxides

picramic acid

picramide

picric acid

picryl chloride

picyrl fluoride

polynitro aliphatic compounds

potassium nitroaminotetrazole

silver acetylide

silver azide

silver stryphnate

silver tetrazene

sodatol

sodium amatol

sodium dinitro-ortho-cresolate

sodium nitrate-potassium

explosive mixtures

sodium picramate

syphnic acid

tetrazene

tetranitrocarbazole

tetrytolhydrazoic acid

trinitroanisole

trinitrobenezene

trinitrobenzoic acid