|  | Industrial Stormwater Pollution Prevention Plan (SWPPP) template NPDES/SDS Industrial Stormwater Permit Doc Type: SWPPP Template |
| --- | --- |

Instructions:  Complete each section of this template to create a Stormwater Pollution Prevention Plan (SWPPP). A SWPPP must be completed before a facility applies for industrial stormwater permit coverage.

Customize the SWPPP to meet the conditions at the facility.

Use discretion about which items need to be addressed. Some sections may not pertain to the facility.

  - Full explanations for each part of this SWPPP are in the 2025-2030 Industrial Stormwater Permit: https://www.pca.state.mn.us/sites/default/files/wq-strm3-102g.pdf found on the Minnesota Pollution Control Agency (MPCA) website at https://www.pca.state.mn.us/business-with-us/industrial-stormwater

The SWPPP is not submitted to the MPCA. It is for your use at the facility. You may add pages with other useful information. Note: Double click on checkboxes to mark as checked.

You may draft a SWPPP using your own format, if you prefer. This template, the SWPPP Checklist (https://www.pca.state.mn.us/sites/default/files/wq-strm3-57.doc), and the permit itself are good guides for what to include in your SWPPP.

Purpose:  A SWPPP is a plan to prevent rain, snow, snowmelt, and runoff – known collectively as stormwater – from being polluted due to contact with your facility’s industrial activities and significant materials that are stored outside. Use the SWPPP to establish what will be implemented at your site and also to keep track of what actions were taken and when.

Resources:

| Click the link | Or search MPCA website http://www.pca.state.mn.us for: |
| --- | --- |
| MPCA Industrial Stormwater General Permit | wq-strm3-102g |
| Template for writing the SWPPP (this document) | wq-strm3-28 |
| Checklist of what to include in the SWPPP | wq-strm3-57 |
| Industrial Stormwater Best Management Plan guidebook. | wq-strm3-26 |
| Inspection form | wq-strm3-56 |
| Mercury Minimization Plan Checklist | wq-strm3-30 |
| Industrial stormwater Special and Impaired waters Search Map | Webpage |

## The above instructions should be deleted before finalizing the SWPPP.

Stormwater Pollution Prevention Plan (SWPPP) for:

[Type facility name here]

[facility address]

This SWPPP is required by the Minnesota Industrial Stormwater Multi-Sector General Permit. Keep this SWPPP at the facility and available for review.

1.	Pollution prevention team

List the person (or title/role) responsible for each of the following duties:

| Name | Stormwater Pollution Prevention Plan duties |
| --- | --- |
|  | Develop and implement the SWPPP |
|  | Install and maintain the BMPs identified in Section 6 of this SWPPP |
|  | Conduct monthly facility inspections |
|  | Collect quarterly stormwater samples |
|  | Submit the Annual Report due March 31 each year |
|  | Conduct annual employee training |
|  | Review the SWPPP annually for updates; modify as needed |
|  | Pay the annual fee |
|  | Other: |
|  | Other: |

2.	Updates

More information is within Part III. Stormwater Pollution Prevention Plan (SWPPP), SWPPP Modification Requirements section.

Review this SWPPP at least once a year to make sure everything is current and updated. Make changes as needed. Record the date the SWPPP was reviewed and summarize any updates made. Make additional copies of this page as needed.

Have any of the following changes happened?

- Construction or change in design, operation, or maintenance at the facility that affects stormwater.

- Inspection identifies deficiencies in the SWPPP or BMPs.

- There was an unauthorized discharge identified from the facility.

- Staffing changes that affect duty assignments or spill response contact information.

- A BMP was determined to be ineffective, or a new one was added to meet applicable water quality standards or to address exceedances of benchmark values.

- Sampling locations have been moved.

- New or different activities or materials come in contact with stormwater.

- Activities from new industry types have been added; check the industry requirements covered in Section 5 of this SWPPP to see if anything needs to be added.

- Impaired waters have been listed within one mile of a facility monitoring location. Search for special and impaired waters on the MPCA’s Industrial Stormwater Special and Impaired Waters Search Tool, an online map search tool.

| Date | Reviewed by | Changes made to SWPPP |
| --- | --- | --- |

3.	Facility description

| Describe what the facility does: |  |
| --- | --- |
| Total acreage of the facility property: |  |
| Acreage that has industrial activities or materials. Exclude areas with no industrial activity and no stormwater discharges flowing through them, such as natural or landscaped areas, employee parking lots, and office buildings: |  |

4.	Site map

More information is within Part III. Stormwater Pollution Prevention Plan (SWPPP), Facility Map section.

Tip:  Make the site map last, since it includes things you will identify as you write the SWPPP.

Create and attach a map of the facility at 1:24,000 or larger scale. This means that every inch on the map represents 2,000 feet or less on the ground. An example of a map at 1:24,000 scale is a U.S. Geological Survey topographic map. It may be easier to have more than one map, or a map and a site diagram.

The maps must show:

|  | Location of the facility. |
| --- | --- |
|  | Impervious surfaces on the property. |
|  | Storm sewer drains/inlets. |
|  | Loading dock drains, including those connected to a storm sewer. |
|  | Directions of stormwater flow indicated by arrows. |
|  | Non-stormwater discharges. Show all that are identified in Section 9 of this SWPPP. |
|  | Locations of industrial activities, materials and structural BMPs inventoried in Section 6 of this SWPPP. |
|  | Surface water body that receives the facility’s stormwater discharges. If the receiving water is beyond the scale of the map, an arrow in the direction of the water and its name is acceptable instead. |
|  | Benchmark monitoring locations. Give each one a unique ID, which will be used when submitting sampling data to the MPCA. For example: BM1, BM2, etc. |
|  | If applicable: Effluent monitoring locations. Give each one a unique ID, which will be used when submitting sampling data to the MPCA. For example: EM1, EM2, etc. |
|  | If applicable: Location of special and/or impaired waters that are within one mile of a monitoring location. Include the name of the water body and the reason it is impaired. The interactive map of special and impaired waters can be found on the MPCA webpage at https://pca-gis02.pca.state.mn.us/ISW/. Indicate which monitoring locations are within a mile of the impaired waters. |
|  | If applicable: Name and location of any water body listed as a special or an outstanding resource value water that is within one mile of a sampling location and receives stormwater from the facility, found in permit section Part X. Additional requirements for discharges to impaired and special (PROHIBITED, RESTRICTED, OTHER) Waters. |

5.	Industry requirements

The industries listed below have extra requirements that must be added to this SWPPP. If this facility’s industry is not listed, there are not extra requirements, see the 2025-2030 permit for sector-specific details: https://www.pca.state.mn.us/sites/default/files/wq-strm3-102g.pdf

| Sector | Description |
| --- | --- |
| A | Timber Products |
| B | Paper and Allied Products Manufacturing |
| C | Chemical and Allied Products Manufacturing |
| D | Asphalt Paving and Roofing Materials and Lubricant Manufacturing |
| E | Glass, Clay, Cement, Concrete and Gypsum Products |
| F | Primary Metals |
| G | Metal Mining (Ore Mining and Dressing) |
| H | Coal Mines and Coal Mining-Related Facilities |
| I | Oil and Gas Extraction and Refining |
| J | Mineral Mining and Dressing |
| K | Hazardous Waste Treatment, Storage, or Disposal Facilities |
| L | Landfills and Land Application Sites |
| M | Automobile Salvage Yards |
| N | Scrap Recycling and Waste Recycling Facilities |
| O | Steam Electric Generating Facilities |
| P | Land Transportation and Warehousing |
| Q | Water Transportation |
| R | Ship and Boat Building and Repair Yards |
| S | Air Transportation |
| T | Treatment Works |
| U | Food and Kindred Products |
| V | Textile Mills, Apparel and Other Fabric Products Manufacturing |
| W | Furniture and Fixtures |
| X | Printing and Publishing |
| Y | Rubber, Miscellaneous Plastic Products and Miscellaneous Manufacturing Industries |
| Z | Leather Tanning and Finishing |
| AA | Fabricated Metal Products |
| AB | Transportation Equipment and Industrial or Commercial Machinery |
| AC | Electronic and Electrical Equipment and Components, Photographic and Optical Goods |

6.	Materials and activities inventory and best management practices (BMPs)

More information is within Part II. Stormwater Control Measures and Part III. Stormwater Pollution Prevention Plan (SWPPP) sections. If flocculants or polymers are added as a BMP (this is uncommon), see the permit for additional requirements.

On the next page, list the activities and materials at the facility that could come in contact with rain, snow, snowmelt, or runoff. Then explain how contact will be limited or prevented. The things done to accomplish this are referred to as best management practices (BMPs). This is the heart of the SWPPP. Tips and examples are listed below.

Note:  If all stormwater contact can be eliminated throughout the entire facility, apply for a no exposure certification from the ISW general permit via e-Services. For more information about no exposure certification requirements, refer to the MPCA ISW webpage “Step 2: Certify for no exposure” at https://www.pca.state.mn.us/business-with-us/step-2-certify-for-no-exposure.

Tips:

Source of pollution:  This can be an activity or a material. Take a walk around the property to see what is exposed to rain, snow, snowmelt, or runoff. Common sources of stormwater pollutants include material loading and unloading areas, fuel dispensers, dust from vents or baghouses, old equipment stored outside, dumpsters or compactors and dirty pallets.

Pollutants:  Safety Data Sheets (SDS) can be helpful for this.

BMP:  Describe how the source of pollution will be protected from rain, snow, snowmelt, or runoff. Ideas for BMPs are on the MPCA industrial stormwater webpage “Step 4: Create SWPPP, choose BMPs” at https://www.pca.state.mn.us/business-with-us/step-4-create-swppp-choose-bmps and in the MPCA’s Industrial stormwater best management plan guidebook: https://www.pca.state.mn.us/sites/default/files/wq-strm3-26.pdf

Structural or non-structural:  Is the BMP structural or non-structural? Structural BMPs are built objects, such as curbs around gas pumps. Non-structural BMPs are common sense practices, such as storing materials indoors or keeping dumpster lids closed. Implement structural BMPs within one year of receiving the permit. Start using non-structural BMPs immediately.

Type:  Address each of the following at least once:

Good housekeeping. Keep exposed areas clean and debris-free. Pick up around dumpsters and loading docks regularly.

Eliminate or reduce exposure. Try to find ways to eliminate exposure to stormwater altogether, such as by moving materials indoors. If exposure cannot be eliminated, try to reduce it, such as by keeping dumpsters closed.

Management of runoff. Look for ways to divert stormwater away from areas that could contribute pollutants or remove pollutants before the stormwater leaves the property. For example, building a curb uphill of a fueling area so stormwater flows around it rather than through it.

Erosion prevention and sediment control. In areas that are easily eroded, such as slopes, garden beds, or gravel parking lots, stabilize the soil and prevent it from being washed off site. Use plants, structures, or other means to keep the soil in place.

Salt storage piles (if present) must be covered or enclosed.

Examples:

Example 1:

Source of pollution:  Scrap metal waste stored in dumpster on the loading dock.

Pollutants:  Zinc, cutting oil

BMP:  Signs are posted to remind everyone to close the dumpster cover tightly after adding scrap, because keeping the cover on keeps rain and snow out. Pat will sweep up debris around the dumpsters weekly to reduce exposure. Pat will also check to make sure dumpsters are not overfilled.

Structural or non-structural: Structural (dumpster covers) and non-structural (cleaning up).

Type:	 Good housekeeping    Eliminates or reduces exposure    Management of runoff

Erosion prevention and sediment control    Salt storage (if present)

Example 2:

Source of pollution:  Stained, dirty, and broken pallets

Pollutants:  Oil, dirt, particles, rust

BMP:  Move indoors, store in southwest corner of receiving area. Shipping clerk will check loading dock daily to make sure dirty pallets are not stacked up outside.

Structural or non-structural:  Non-structural

Type:	 Good housekeeping    Eliminates or reduces exposure    Management of runoff

Erosion prevention and sediment control    Salt storage (if present)

Stormwater management for [facility name]

- Source of pollution:

Pollutants:

BMP:

Structural or non-structural:

Type:	 Good housekeeping    Eliminates or reduces exposure    Management of runoff

Erosion prevention and sediment control    Salt storage (if present)

- Source of pollution:

Pollutants:

BMP:

Structural or non-structural:

Type:	 Good housekeeping    Eliminates or reduces exposure    Management of runoff

Erosion prevention and sediment control    Salt storage (if present)

- Source of pollution:

Pollutants:

BMP:

Structural or non-structural:

Type:	 Good housekeeping    Eliminates or reduces exposure    Management of runoff

Erosion prevention and sediment control    Salt storage (if present)

- Source of pollution:

Pollutants:

BMP:

Structural or non-structural:

Type:	 Good housekeeping    Eliminates or reduces exposure    Management of runoff

Erosion prevention and sediment control    Salt storage (if present)

- Source of pollution:

Pollutants:

BMP:

Structural or non-structural:

Type:	 Good housekeeping    Eliminates or reduces exposure    Management of runoff

Erosion prevention and sediment control    Salt storage (if present)

- Source of pollution:

Pollutants:

BMP:

Structural or non-structural:

Type:	 Good housekeeping    Eliminates or reduces exposure    Management of runoff

Erosion prevention and sediment control    Salt storage (if present)

- Source of pollution:

Pollutants:

BMP:

Structural or non-structural:

Type:	 Good housekeeping    Eliminates or reduces exposure    Management of runoff

Erosion prevention and sediment control    Salt storage (if present)

- Source of pollution:

Pollutants:

BMP:

Structural or non-structural:

Type:	 Good housekeeping    Eliminates or reduces exposure    Management of runoff

Erosion prevention and sediment control    Salt storage (if present)

Add more if needed.

7.	Inspections

More information is within Part II. Stormwater Control Measures, Facility Inspection Requirements section.

Conduct and document inspections monthly. Facilities are required to conduct either one or two runoff inspections per year, depending on their sector. Review permit requirements in Part VIII. Sector-specific requirements to see which frequency of inspections apply to your facility.

Use the inspection forms available on the MPCA webpage - https://www.pca.state.mn.us/business-with-us/step-6-meet-requirements or develop your own inspection forms based on section 19.1 of the permit and any sector-specific requirements in Part VIII Sector-specific requirements.

Attach the inspection forms here or store them with the SWPPP.

8.	BMP Maintenance Log

More information is within Part II. Stormwater Control Measures, BMP Maintenance section.

If monthly inspections find BMPs that are not working properly, replace or repair them within seven calendar days. If the work cannot be done in seven calendar days, put effective backup BMPs in place (temporarily or permanently) within 48 hours of discovery. Record the maintenance done. Explain replacements or repairs that take longer than seven days.

| Date of maintenance | Inspector | Summary of maintenance performed |
| --- | --- | --- |
| Example: 4/16/26 | Chris L. | Dumpster lid was damaged in storm. Large chunk missing and does not close properly. Requested a replacement dumpster from hauler. Will be delivered 4/18/26. |

9.	Non-stormwater discharges

The permit allows some types of water to be discharged with stormwater even though they are not stormwater.

List all surface discharges from the property that are anything other than stormwater or snowmelt.

Evaluate whether they are allowed or prohibited. Discharges that the permit allows and prohibits have been reprinted on the next page.

Eliminate prohibited discharges. Minimize erosion and limit the discharge of dirt and sediment from allowed discharges.

| Discharge, source and source location | Is it allowed or prohibited? | How did you determine if it was allowed or prohibited? | Action taken | Date of evaluation | Which monitoring locations were checked for this discharge? |
| --- | --- | --- | --- | --- | --- |
| Example: Air conditioner condensate from rooftop unit on main building. | Allowed | A/C condensate is listed in permit as an allowed discharge. | Added a piece of stone under the spout to keep lawn from eroding. Added to monthly inspection list to check and make sure the water is clear. | 10/1/26 | All benchmark locations |

Allowed discharges:

- Emergency fire-fighting activities

- Fire hydrant and sprinkler system flushing except those which contain or have contained per-and polyfluoroalkyl substances (PFAS)

- Potable water line flushing

- Uncontaminated condensate from air conditioners, coolers, compressors, or the outside storage of refrigerated gases or liquids

- Landscape watering if all pesticides, herbicides, and fertilizers have been applied in accordance with manufacturer’s instructions

- Pavement wash water if no detergents are used and there are no spills or leaks of pollutants such as salt, fertilizer, toxic materials, or hazardous materials, or all spilled material has been removed

- Water from washing the outside of a building if no detergents, solvents, or degreasers are used

- Uncontaminated groundwater or spring water

- Foundation or footing drains if water is not contaminated

- Incidental windblown mist from cooling towers that collects on rooftops or adjacent portions of the facility

- Discharges authorized by a separate permit

Prohibited discharges:

- Water from washing commercial equipment or vehicles

- Drainage from floor drains in process areas

- Non-contact cooling water

- Discharges prohibited in the sector-specific requirements of the permit

- Domestic, industrial and process wastewater

- Spills of any substance that may cause water pollution

- PFAS-containing foam

- Biosolids

- Stormwater discharges from construction activity

- Discharges to impaired waters if the Total Maximum Daily Load (TMDL) report for that waterbody prohibits discharges

- Discharges prohibited in Minn. R. 7050.0180, subp. 3, 4 and 5. This includes certain discharges to the Boundary Waters Canoe Area Wilderness, Voyageur's National Park, parts of Lake Superior, parts of the Kettle River and the Rum River and Minnesota Department of Natural Resources Scientific and Natural Areas

- Discharges from industrial activity areas that are more than 90 acres in size

10.	Equipment Preventative Maintenance Program

If this facility already has an equipment preventative maintenance program and/or an Operation and Maintenance Manual that addresses the requirements listed below, it can be attached here or incorporated by reference instead of rewriting the procedures. More information is within Part II. Stormwater Control Measures, Equipment Preventative Maintenance section.

The program needs to include the following:

Regular inspection of industrial equipment

Regular maintenance of industrial equipment and systems

Repair of industrial equipment and systems

The inspections must identify conditions that could cause breakdowns or failures, which may result in leaks, spills, and other releases (e.g., hydraulic leaks, torn bag-house filters, etc.), and the discharge of pollutants to stormwater.

11.	Spill prevention and response

If this facility already has a spill response plan that addresses the requirements listed below, it can be attached here or incorporated by reference instead of rewriting the procedures. More information is within Part II. Stormwater Control Measures, Spill Prevention and Response Requirements section.

Note: The Permittee shall immediately notify the Minnesota Department of Public Safety Duty Officer at 800-422-0798 (toll free) or 651-649-5451 (metro area) per Minn. Stat. 115.061 of any spill or discharge that has the potential to pollute waters of the state.

If this facility stores, transfers, or uses solid or liquid materials that could pollute stormwater, develop a procedure to prevent spills and respond to those that happen. Think especially of materials that are delivered or stored in bulk, such as gasoline, diesel, flour, sawdust or sand.

Include the following:

Which materials could spill or leak;

Areas where spills and leaks could occur;

How the possibility and impact of spills or leaks will be reduced through handling procedures and storage requirements;

How any spilled or leaked materials will be cleaned up in a rapid and thorough manner including identification of cleanup materials, equipment, and procedures;

Monitoring locations and surface waters that would be affected by spills and leaks;

Contact information for staff members, emergency agencies, and regulatory agencies, including the Minnesota Duty Officer;

All actions that will be taken to prevent PFAS foam-containing firefighting runoff water from entering stormwater systems or flowing to surface waters;

Records of spills and leaks, including quantities; and

How the spill prevention and response plan shall be made available for appropriate facility personnel.

| Date of spill/ leak (mm/dd/yyyy) | Material spilled/ leaked | Volume released | Impacted media (soil/water/sewer/monitoring location) | Action taken to recover and mitigate |
| --- | --- | --- | --- | --- |
| Example: 7/12/2026 | Hydraulic fluid from forklift | 6 gallons | Released to asphalt, no sampling locations or surface waters impacted | Placed floor dry on spill, scooped up and disposed of properly. Duty Officer notified, report #210641 |

12.	Employee training program

Make additional copies of this page for each training event. More information is within Part II. Stormwater Control Measures, Employee Training Program section.

For this permit to successfully protect Minnesota’s waters, employees need to know what to do and why.

Train the employees who use or update the SWPPP, work with the materials or activities that are exposed to stormwater, do inspections or maintenance of the BMPs, and collect stormwater samples. The topics to cover are listed below.

Train annually, or more frequently if appropriate.

- How often will stormwater training be conducted?

Attached training documents to this SWPPP.

Trainer:

Trainer’s employer:

Topics covered:

Purpose of the SWPPP

What is in this the SWPPP

Use and maintenance of BMPs

How to do a monthly inspection

How and where stormwater samples are collected

Spill prevention and response

Other:

| Date (month/day/year) | Employee name |
| --- | --- |

13.	Mercury minimization

More information is within Part II. Stormwater Control Measures, Mercury Minimization Plan section.

If mercury or mercury-containing devices are exposed to stormwater at this facility, describe the mercury sources or devices and how they will be managed to eliminate exposure to precipitation and stormwater runoff. Use the Mercury Minimization Plan Checklist to do this.

Sector M: Automobile Salvage Yards and Sector N: Scrap Recycling and Waste Recycling Facilities have additional Mercury Minimization Plan requirements. Further details are within Part VIII. Sector-specific Requirements section.

No mercury sources or devices exposed to stormwater on site. No Mercury Minimization Plan required.

Mercury present on site. Mercury Minimization Plan attached.

14.	PFAS Requirements

Facilities with a primary SIC Code listed in Appendix D of the permit shall abide by all requirements and monitor for PFAS using the procedures outlined in Part XI Per-and Polyfluoroalkyl Substances (PFAS) Monitoring and Reporting Requirements section of the permit. This includes, but is not limited to:

Develop and implement a PFAS Stormwater Monitoring Plan for the facility. The plan must be kept with the facility’s SWPPP, and:

- Contain all items listed within the SWPPP and PFAS Stormwater Monitoring Plan Requirements section of the permit.

- Be reviewed and updated annually

If required, complete and begin implementing a PFAS source and exposure reduction plan (SERP) utilizing the MPCA’s current SERP template. The SERP template and other PFAS and industrial stormwater guidance can be found on the MPCA’s webpage - https://www.pca.state.mn.us/business-with-us/industrial-stormwater-and-pfas

15.	Mobile operations

If this facility has mobile operations, stormwater pollution must be prevented at those locations, too. However, it is not necessary to develop an entirely new SWPPP. Think of it as a mini-SWPPP. Keep a copy of the ‘mini-SWPPP’ at the location(s) where the mobile operations occur.

Complete new versions of these sections of the SWPPP template with the mobile operations in mind:

Section 5. Industry requirements

Section 6. Materials and activities inventory

Section 7. Inspections

Section 8. Maintenance

Section 9. Non-stormwater discharges

Section 10. Equipment Preventative Maintenance

Section 11. Spill prevention and response

Section 12. Employee training program

Section 13. Mercury minimization

Section 14. PFAS requirements