|  | SWPPP Checklist Construction Stormwater Permit Program Doc Type: Stormwater Pollution Prevention Plan (SWPPP) |
| --- | --- |

| ## C000 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

Background: This checklist is used by Minnesota Pollution Control Agency (MPCA) staff for Stormwater Pollution Prevention Plan (SWPPP) reviews. It is provided as an additional resource intended for SWPPP designers for construction projects to assure all required elements of a SWPPP are included. Use of this checklist will help you to determine if your SWPPP is complete, though not all checklist items are applicable to all projects. This checklist can be used for all size projects; however, the guidance document “Stormwater Compliance Assistance Toolkit for Small Construction Operators,” contains a SWPPP template designed specifically for small site projects. This guidance is available on the MPCA Construction Stormwater webpage at: http://www.pca.state.mn.us/wfhya5b.

Note - This checklist is for your information and use is voluntary. The checklist does not need to be returned to the MPCA.

## Review Information

| Applicant: |  |  |  | Project name: |  |
| --- | --- | --- | --- | --- | --- |
| Application date: |  |  | Reviewer name: |  |  |

Reason for review:

Yes N/A Notes

Mandatory (over 50 acres and discharging to a special or impaired water)

Random audit

Enforcement case

| Case lead: |  |
| --- | --- |

SWPPP contains a combination of:

Yes	N/A

Narrative

Plan sheets

Standard detail sheets (where appropriate)

## SWPPP Information (does the Narrative contain the following)

Yes	N/A

Describe the nature of the construction activity?

Address the potential for a discharge of sediment and/or other potential pollutants from the site?

Identify the person who will oversee the SWPPP implementation?

Identify the entity responsible for long term Operations and Maintenance (O&M) of the permanent stormwater management system?

List the chain of responsibility for SWPPP implementation for all operators on the site?

Identify the training requirements are satisfied.

Describe installation timing for all Erosion Sediment Control (ESC) Best Management Practices (BMPs)?

Describe procedures to establish additional temporary ESC BMPs as necessary for site conditions?

Describe final stabilization methods for all exposed areas? (may be in narrative or on plan sheets)

Identify stormwater management measures needed to mitigate impacts identified as a result of environmental, historical, archaeological, or rare species reviews conducted for the project?

Identify additional measures being taken to protect Drinking Water Supply Management Areas?

If site discharges to special water or impaired reach, identify any site areas discharging to the special or impaired reach?

| Comments: |  |
| --- | --- |

## Do plan sheets identify:

Yes	N/A

Existing and final grades.

Locations and types of all temporary and permanent ESC BMPs.

Stormwater flow directions and surface water divides for all pre- and post-construction drainage areas.

## Do plan sheets identify (continued):

Yes	N/A

Impervious areas.

Soil types.

Locations of areas not to be disturbed.

Tabulated quantities of all erosion prevention and sediment control BMPs.

Limits of construction phases.

Locations of all wetlands and surface waters that will receive pre- or post-construction site runoff. (If they do not  fit on the plan sheets, use an arrow to note the direction and distance.

| Comments: |  |
| --- | --- |

## Standard plates or specifications:

Yes	N/A

Are standard plates or specifications included where appropriate?

## Part III - Stormwater Discharge Design Requirements

Yes	N/A

Are Temporary Sediment Basins required on site? (10 acres draining to common location)

If Yes, are they:

Yes	N/A

Adequately sized – 2-year, 24-hour storm, minimum 1,800 feet3/acre; or no calculative minimum 3,600ft3/acre?

Designed to prevent short circuiting?

Are outlets designed to remove floating debris?

Are outlets designed to allow complete drawdown?

Do outlets have energy dissipation?

Have a stabilized emergency spillway?

| Comments: |  |
| --- | --- |

Yes	N/A

Permanent Stormwater Management System

Yes	N/A

Is calculation of new impervious surface included in SWPPP?

Are calculations for permanent stormwater management system included (water quality volume of one-half inch of runoff)?

Are there areas of the project where typical treatment methods are not feasible? (Up to one percent of project size or three cumulative acres or proximity to bedrock or road projects lacking of right of way.)

If yes, has effort been made to provide some treatment using alternatives?

Yes	N/A

Grassed swales

Smaller ponds

Grit chambers

| Comments: |  |
| --- | --- |

Which method of permanent stormwater treatment has been selected?

Yes	N/A

Wet sedimentation basin:

Yes	N/A

Permanent volume of 1,800 feet3 below outlet pipe for each acre draining

Minimum depth of 3 feet; maximum depth of 10 feet

Configured so scour or resuspension is minimized

Water quality volume is one-half inch of runoff from new impervious surfaces

Basin outlets designed to discharge at less than 5.66 cubic feet per second (cfs) per acre of pond

Basin outlets designed to prevent short circuiting

Basin outlets designed to prevent discharge of floatables

Stabilized emergency overflow

Is adequate maintenance access provided

| Comments: |  |
| --- | --- |

Yes	N/A

Infiltration/filtration:

Yes	N/A

Is infiltration/filtration appropriate to the site and land uses?

Is infiltration system not excavated to final grade until drainage area constructed and stabilized?

Are rigorous sediment and erosion controls used to keep sediment and runoff away?

Is a pretreatment device used?

Is the system sufficient to infiltrate or filter the appropriate water quality volume?

Can water quality volume be discharged in 48 hours or less?

If not, are they routed through stabilized discharge point?

Is there a way to visually verify the system is operating as designed?

Has appropriate testing been conducted to ensure a minimum of 3 feet of separation?

Are calculations and computer model results included to demonstrate the design adequacy of the infiltration system?

Is adequate maintenance access provided?

Does the maintenance plan identify who will perform future maintenance?

| Comments: |  |
| --- | --- |

Yes	N/A

Regional ponds:

Yes	N/A

Is written authorization from owner of regional pond included in SWPPP?

Is there no significant degradation of waterways between project and regional pond?

Does regional pond design conform to the permit requirements for wet sedimentation basin?

| Comments: |  |
| --- | --- |

Yes	N/A

Combination of practices:

Yes	N/A

Is the entire water quality volume be accounted for?

Are computer models and/or calculations included in the SWPPP?

| Comments: |  |
| --- | --- |

Yes	N/A

Alternative method:

Yes	N/A

SWPPP, including the Alternative Method documentation, to MPCA for review and approval at least 90 days prior to the proposed starting date of construction activity.

| Comments: |  |
| --- | --- |

## Part IV – Construction Activity Requirements

Yes	N/A

Addresses erosion prevention measures:

Yes	N/A

Are areas not to be disturbed delineated on plans?

Has appropriate construction phasing been implemented?

Do exposed soils have erosion protection/cover within 14 days?

Are wetted perimeters of ditches stabilized within 200 feet of surface water within 24 hrs?

Do pipe outlets have energy dissipation within 24 hours of connecting?

| Comments: |  |
| --- | --- |

Yes	N/A

Addresses sediment control measures:

Yes	N/A

Are slopes with a 3:1 grade broken up into lengths less than 75 feet?

Are sediment control practices established on down gradient perimeters?

Are all inlets protected?

Do stockpiles have sediment control and placed in areas away from surface waters?

Do construction site entrances minimize street tracking?

| Comments: |  |
| --- | --- |

Yes	N/A

Addresses dewatering and basin draining:

Yes	N/A

Is there a plan in place for dewatering so as to not cause nuisance conditions, erosion, or inundation?

| Comments: |  |
| --- | --- |

Yes	N/A

Addresses inspections and maintenance:

Yes	N/A

Identifies the person who will oversee the BMP inspection and maintenance?

Inspections performed once every 7 days

Inspections performed within 24 hours of a rain event greater than 0.5 in/24 hours

Inspection and Maintenance records include:

Yes	N/A

Date and time of inspection

Name of person(s) conducting inspections

Finding of inspections and recommendations for corrective actions

Date and amount of rainfall events greater than 0.5 in/24 hours

Yes	N/A

Maintenance performed

Yes	N/A

Silt fence repaired/replaced/supplemented when nonfunctional, or one-third full; within 24 hours

Sediment basins drained and sediment removed when reaches one-half storage volume; within 72 hours

Sediment removed from surface waters within 7 days

Construction site exits inspected, tracked sediment removed within 24 hours

| Comments: |  |
| --- | --- |

Yes	N/A

Addresses pollution prevention management measures:

Yes	N/A

Solid waste disposed properly; comply with MPCA requirements

Hazardous waste stored (secondary containment, restricted access) and disposed in compliance with MPCA requirements

External washing of vehicles limited. Runoff contained and waste properly disposed

No engine degreasing allowed on site

Concrete washout provided (container or lined)

| Comments: |  |
| --- | --- |

Yes	N/A

Addresses final stabilization:

Yes	N/A

Stabilization by uniform perennial vegetative cover (70 percent density)

Drainage ditches stabilized

All temporary synthetic and structural BMPs removed

Clean out sediment from conveyances and sedimentation basins (return to design capacity)

If residential – distribute homeowner factsheet

Submit Notice of Termination (NOT)

| Comments: |  |
| --- | --- |

Requirements of Appendix A

Yes	N/A

Does this site drain to a discharge point on the project that is within one mile of a Special or Impaired Water?

| Yes N/A Which type of special water? | BMP category |
| --- | --- |
| Wilderness Areas | C.1, C.2, C.3, C.4 |
| Mississippi River | C.1, C.2, C.3 |
| Scenic or Recreational river | C.1, C.2, C.3 |
| Lake Superior | C.1, C.2, C.3 |
| Lake Trout Lakes | C.1, C.2, C.3, C.4 |
| Trout Lakes | C.1, C.2, C.3, C.4 |
| Scientific and Natural areas | C.1, C.2, C.3, C.4 |
| Trout Streams | C.1, C.2, C.3, C.5 |

| Yes N/A Impaired water | BMP category |
| --- | --- |
| TMDL and/or WLA not yet approved | C.1, C.2 |
| Approved TMDL and WLA | BMPs in TMDL |

TMDL = Total Maximum Daily Loads
WLA = Waste Load Allocations

| BMP category | Requirement |  |
| --- | --- | --- |
| Yes N/A C.1 | All soils protected in seven days/provide temp basin for five acres draining to common location. |  |
| C.2 | Treat water quality volume of one inch of runoff with one-half inch infiltrated, or |  |
|  | Infiltration not possible for the following reason(s): |  |
|  | Yes N/A Lack of depth to groundwater D type soils Runoff from areas with known contaminants (stormwater hotspot) Proximity to bedrock |  |
|  | Other: |  |
| C.3 | Maintain buffer zone of 100 linear feet from Special Water. |  |
| C.4 | Post project volume/rate control. |  |
| C.5 | Temperature controls. |  |

| Comments: |  |
| --- | --- |

| Does this site discharge to wetlands? |  |
| --- | --- |
|  | Yes N/A Has the wetland mitigation sequence been followed? |

| Comments: |  |
| --- | --- |