|  | Laboratory Data Review Checklist Doc Type: Data Review |
| --- | --- |

Instructions:  The following is the Minnesota Pollution Control Agency’s (MPCA) informal checklist that may be used to review data. The information follows the general format of the National Functional Guidelines which is the primary data review tool used in the U.S. Environmental Protection Agency’s Contract Laboratory Program for Superfund analytical work. This checklist should be used in conjunction with the Laboratory Data Checklist Guidance (p-eao-11a): http://www.pca.state.mn.us/index.php/view-document.html?gid=16113. Also see the MPCA Laboratory Quality Control (QC) and Data Policy: http://www.pca.state.mn.us/index.php/view-document.html?gid=16288.

## Project Information

| Project name: |  |  | Laboratory: |  |  |
| --- | --- | --- | --- | --- | --- |
| Work order number: |  |  | Report date (mm/dd/yyyy): |  |  |

| 1. | Preservation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | For help with this section on holding times, containers and preservatives, refer to the Minnesota Department of Health’s website at: http://www.health.state.mn.us/divs/phl/environmental/handbook/internet/envhandbook.html. |  |  |  |  |  |  |
|  | Questions |  |  | Yes | No | N/A | Comments |
|  | a. | Is there a chain of custody (COC) with the report? |  |  |  |  |  |
|  | b. | Is there a sample condition form with the report? |  |  |  |  |  |
|  | c. | Were there samples requiring preservation? |  |  |  |  |  |
|  |  | i. | If so, were they properly preserved? |  |  |  |  |
|  |  | ii. | Were they received on ice? |  |  |  |  |
|  | d. | Were samples received in the correct containers? |  |  |  |  |  |
|  |  | i. | Was there enough sample volume/weight to complete all requested analyses? |  |  |  |  |
|  |  | ii. | Was there enough extra sample collected to complete method required batch QC? |  |  |  |  |
|  | e. | Were samples received with adequate holding time for sample prep for all requested analyses? |  |  |  |  |  |
|  | f. | Are there notes about sample condition or holding time issues on the COC? Explain impact. |  |  |  |  |  |
|  | g. | Is there narration or data qualifiers within the report about sample condition or holding time issues? Explain impact. |  |  |  |  |  |

| 2. | Calibration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Question |  | Yes | No | N/A | Comments |
|  | a. | Do the report narrative or data qualifiers indicate calibration problems for any analyses? If yes, explain the data impact. |  |  |  |  |

| 3. | Blanks |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Question |  |  | Yes | No | N/A | Comments |
|  | a. | Do any of the analyses contain samples for field or trip blanks? |  |  |  |  |  |
|  |  | i. | If yes, are there target analytes present above the reporting limit? |  |  |  |  |
|  |  | ii. | If yes, are the same compounds also present in the samples? Explain possible impact. |  |  |  |  |
|  | b. | Do method blanks for any analyses contain target analytes above the reporting limit? |  |  |  |  |  |
|  |  | i. | If yes, are the same compounds present in the samples? |  |  |  |  |
|  |  | ii. | Is the amount of target analyte in the blank more than 1/10th of that in the sample(s)? Explain the possible impact on sample results. |  |  |  |  |
| 4. | Surrogates |  |  |  |  |  |  |
|  | Question |  |  | Yes | No | N/A | Comments |
|  | a. | Are there organic analyses that contain surrogate compounds? |  |  |  |  |  |
|  | b. | Are the lab recovery limits specified on the report? |  |  |  |  |  |
|  |  | i. | Do the lab limits seem reasonable when compared with the suggested guidelines in the MPCA QC Policy? |  |  |  |  |
|  | c. | Are there surrogates outside lab limits? (These should have a data qualifier) |  |  |  |  |  |
|  |  | i. | If yes, are the surrogates above the lab limits? |  |  |  |  |
|  |  | ii. | Below the lab limits? |  |  |  |  |
|  |  | iii. | Explain what this could mean for the affected samples. |  |  |  |  |
| 5. | Laboratory Control Sample/Laboratory Control Sample Duplicate (LCS/LCSD) |  |  |  |  |  |  |
|  | Question |  |  | Yes | No | N/A | Comments |
|  | a. | Are there LCS/LCSD samples present for the reported analyses? (An LCS alone is acceptable if there is an Matrix Spike/Matrix Spike Duplicate [MS/MSD] or sample/sample dup for precision.) |  |  |  |  |  |
|  |  | i. | If so, do the lab limits seem reasonable compared to the suggested guidelines in the MPCA QC Policy? |  |  |  |  |
|  | b. | Are there LCS/LCSD compounds outside lab limits? (These should have a data qualifier.) |  |  |  |  |  |
|  |  | i. | If yes, are the analytes above the lab limits? |  |  |  |  |
|  |  | ii. | Below the lab limits? |  |  |  |  |
|  |  | iii. | Are all samples in the preparation batch also flagged for the same analyte(s)? |  |  |  |  |
|  |  | iv. | Explain what this could mean for the affected samples. |  |  |  |  |

| 6. | Matrix Spike/Matrix Spike Duplicate/Sample Duplicate (MS/MSD/Dup) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Question |  |  |  | Yes | No | N/A | Comments |
|  | a. | Do the analytical methods used require an MS and/or MSD? If no, skip to 6.b. |  |  |  |  |  |  |
|  |  | i. | Have the required matrix spikes been prepared and reported? |  |  |  |  |  |
|  |  | ii. | If no, is there and explanation in the report as to why? |  |  |  |  |  |
|  |  | iii. | Did the lab process an alternate spiked sample (such as LCSD) instead? |  |  |  |  |  |
|  |  | iv. | Are the lab limits specified on the report? |  |  |  |  |  |
|  |  | v. | Do the limits seem reasonable when compared to the suggested guidelines in the MPCA QC Policy? |  |  |  |  |  |
|  |  | vi. | Are there compounds outside the lab limits? |  |  |  |  |  |
|  |  |  | 1. | If yes, are the analytes above the lab limits? |  |  |  |  |
|  |  |  | 2. | Below the lab limits? |  |  |  |  |
|  |  |  | 3. | Is the source sample also flagged for compounds outside lab limits? |  |  |  |  |
|  | b. | Is a sample duplicate reported for the analytical method(s)? If no, skip to 6.c. |  |  |  |  |  |  |
|  |  | i. | Is the RPD for the duplicate pair within the lab limits? |  |  |  |  |  |
|  |  | ii. | If no, has the associated source sample been flagged? |  |  |  |  |  |
|  | c. | What is the impact of failed QC on this project? |  |  |  |  |  |  |
| 7. | Method Detection Limits/Report Limits |  |  |  |  |  |  |  |
|  | Question |  |  |  | Yes | No | N/A | Comments |
|  | a. | Are reporting and/or method detection limits clearly listed on the report for all analyses? (may also be called quantitation limits) |  |  |  |  |  |  |

Additional comments on report: