|  | Emergency Response Plan NPDES/SDS Permit Program Feedlot Program Doc Type: Permit Application |
| --- | --- |

## Instructions:  This Emergency Response Plan is to be used in case of an emergency spill, leak, or failure at the production facility or land application area. This plan is incorporated into the National Pollutant Discharge Elimination System (NPDES)/State Disposal System (SDS) Permit and submitted to the Minnesota Pollution Control Agency (MCPA).

| ## Facility name: |  |  | ## Feedlot registration no.: |  |
| --- | --- | --- | --- | --- |
| ## Owner/Operator name: |  |  | ## Feedlot permit no.: |  |

List of critical phone numbers and contacts

|  | Contact person (or Company) | Phone number |  |
| --- | --- | --- | --- |
| Emergency contacts |  |  |  |
| - Fire/Ambulance | ------------------------------------------ | 911 |  |
| - County Sheriff |  |  |  |
| Agency contacts |  |  |  |
| - Minnesota Duty Officer | ------------------------------------------ | 1-800-422-0798 | Provide the MN Duty Officer: - Your contact information - Spill location, date and time? - Spill type and spill amount? - Surface water or field tile impacted? - Progress made to clean up or contain the spilled material? |
| - MPCA Field Office |  |  |  |
| - County Feedlot Officer |  |  |  |
| - County Health Department |  |  |  |
| Other contacts |  |  |  |
| - Insurance company |  |  |  |
| - Gopher State One Call | ------------------------------------------ | 1-800-252-1166 |  |
| Liquid manure cleanup equipment and supplies (Local vendors for cleanup assistance) |  |  |  |
| - Manure pumper |  |  |  |
| - Earth moving equipment |  |  |  |
| - Tiling equipment |  |  |  |
| - Containment/Absorption materials (hay, straw, cornstalks, sawdust) |  |  |  |
| Solid manure cleanup equipment and supplies (Local vendors for cleanup assistance) |  |  |  |
| - Manure loading equipment |  |  |  |

| Liquid Manure Emergency Response 1. Immediately stop the source of the leak or spill: - Turn off pumps or valves - Clamp hoses or park tractor on hoses 2. Contain the spilled manure: - Use skid loader or tractor with blade to make berms - Install bale checks and block downstream culverts - Insert sleeves around tile intakes (or plug/cap) - Use tillage equipment to work ground ahead of spill - Use absorptive materials 3. Make necessary phone calls as listed in the chart above: - Notify Minnesota Duty Officer at 1-800-422-0798 - Notify sheriff’s office if spilled on public roads or right-of-ways for traffic control 4. Cleanup: - Clean up spill immediately from road and roadside - Clean up all material, including the contaminated soil, as soon as possible by pumping, scraping, or by other means - Land apply manure at agronomic rates or place manure back in the manure storage area - Follow recommendations of MPCA staff and/or County Feedlot Officers - Restore site to its original conditions 5. Document your actions: - Keep records of all actions related to the spill and follow up activities | Solid Manure Emergency Response - 1. Make necessary phone calls as listed in the chart above: - Notify Minnesota Duty Officer at 1-800-422-0798 - Notify sheriff’s office if spilled on public roads or right-of-ways for traffic control - 2. Cleanup*: - Clean up spill immediately from road and roadside - Clean up all material, including the contaminated soil, as soon as possible by scraping, or by other means - Land apply manure at agronomic rates or place manure back in the solid manure storage area/stockpile - Follow recommendations of MPCA staff and/or County Feedlot Officers - Restore site to its original conditions - 3. Document your actions: - Keep records of all actions related to the spill and follow up activities - *If rain is expected prior to cleanup; actions need to be taken to contain manure contaminated runoff from discharging. (See # 2 under Liquid Manure Emergency Response) |
| --- | --- |

A detailed site map should be displayed on site to assist employees identify sensitive receptors (surface water, wells, tile intakes, etc).