# Chapter 5

# Manure Management Plans

What is a Manure Management Plan?

A manure management plan (MMP) shows how manure generated at a feedlot facility will be used during the upcoming cropping year(s) in a way that maximizes the benefits of applying manure to cropland, meets all rules and regulations, and protects surface and ground water quality.

- Who needs an MMP?

- MMPs are required by the Minnesota Pollution Control Agency (MPCA) as specified in Minnesota Rules Chapter 7020.2225 when:

- A permit application is submitted from an operation with 100 or more animal units (AU) (permit types include NPDES, SDS, Interim or Construction Short Form); OR

- Manure from a feedlot capable of holding 300 or more AU is applied by someone other than a certified animal waste technician; OR

- Where otherwise required through a county ordinance.

Submitting MMPs

Producers do not need to submit their plans for review unless specifically requested by the MPCA or a County Feedlot Officer, or when applying for a permit. However, you may be asked to show your updated manure management plan and associated records when your feedlot is inspected.

Updating MMPs

Manure management plans must be kept up-to-date. Review the plan each year to reflect changes in crop rotation, manure nutrient levels, changes in manure application methods, and other factors that contribute to the nutrient availability of the manure and the crop nutrient needs.

Contents

Page

2		5.1    Types of Manure Management Plans

4		5.2    Manure Management Plan Writing – Computer Programs, Booklets

5		5.3    Minnesota Requirements for Land Application of Manure

7		5.4    County Requirements for Land Application of Manure

7		5.5    Keeping Records of Land Application

7		5.6    Reviewing Manure Management Plans

## Types of Manure Management Plans

There are three different types of plans required under Minnesota feedlot rules.   The Natural Resources Conservation Service (NRCS) also requires a nutrient management plan for projects involving cost-share money (See table 5-1 for a comparison of different MMPs with NRCS nutrient management plans).

Of the three plans required under Minnesota feedlot rules, the first two types of plans are similar and the requirements for these plans are found in the MPCA fact sheet “Manure Management Plans and Checklist”  www.pca.state.mn.us/publications/wq-f8-07.doc

- Non-NPDES permitted sites and manure ownership is NOT transferred

- NPDES permitted sites and manure ownership is NOT transferred

- MMP for manure that is sold or otherwise considered transferred ownership

- Determining ownership of manure

- MMP and record-keeping requirements are different when manure ownership is transferred.  Manure ownership is not considered to be transferred (i.e. feedlot owner/operator retains ownership) when the answer to either of the following questions is Yes.

1.	Is manure from the feedlot facility applied onto land that is owned, leased, or    rented by the feedlot owner/operator?

Yes   No

- For manure application sites not owned, leased, or rented by the feedlot owner/operator; have you as the feedlot owner/operator or employees working under your direction been given control of the crop and nutrient planning decisions, including planning for manure application rates, timing and methods?

Yes  No

If the answer to either of the above questions is “Yes”, then manure ownership is not transferred. Refer to the MPCA guidelines in the fact sheet:  “Manure Management Plan Requirements”.  If the answer to both questions was “No”, then ownership of manure is transferred.  For transferred ownership of manure, refer to the MPCA guidelines “Manure Management Plan Requirements when Ownership of Manure is Transferred”   www.pca.state.mn.us/publications/wq-f8-12.doc.

- If only a portion of the manure is considered to have transferred ownership, then use the guidelines on transferred ownership for the portion which is sold, and develop the more comprehensive MMP for the manure that does not have transferred ownership.

Helpful tools and information

Transferred ownership

- MMP Plan Requirements when Ownership of Manure is Transferred

- Land Application Agreement form   www.pca.state.mn.us/publications/feedlots-landappagreementform.pdf

- “Guidelines for determining whether manure ownership is  transferred”

Non-transferred ownership

  - Manure Management Plan Requirements and checklist

Table 5-1   Comparison of MPCA and NRCS Manure, nutrient and feedlot management plans

| Long term/strategic | MPCA MMP non-NPDES | MPCA MMP for NPDES | NRCS nutrient mgmt plan |
| --- | --- | --- | --- |
| Field maps | X | X | X |
| Soil survey maps |  |  | X |
| Planned crop rotations | X | X | X |
| Livestock type and numbers | X | X | X |
| Manure storage and handling | X | X | X |
| Soil and water conservation |  | X | X |
| Timing of application | X | X | X |
| Sensitive area ID, setbacks, & management | X | X | X |
| Field risk assessment | X | X | X |
| Phosphorus mgmt on extremely high P soils | X | X | X |
| Manure and soil sampling practices | X | X | X |
| Winter spreading locations and management | X | X | X |
| Operation and maintenance plan including records & safety |  | X | X |
| Annual – planned nutrient applications |  |  |  |
| Rates for each crop or field situation | X | X | X |
| Commercial fertilizer N & P on manured fields | X | X | X* |
| Commercial fertilizer N & P on non-manured fields |  |  | X* |
| Potassium from manure, soil & fertilizer |  |  | X* |
| Basis for rates – yields, N credits, soil test, crop needs | X | X | X |
| Feedlot management plans |  |  |  |
| Emergency response plan |  | X | X CNMP only |
| Dead animal management plan |  | X | X CNMP only |
| Existing storage and handling evaluation |  | X | X CNMP only |
| Air emissions plan |  | X |  |

X* - includes timing and form of commercial fertilizer

## Manure Management Plan Writing – Computer Programs, Booklets

Several tools are available for developing the nutrient management aspects of a MMP. The computer software and hand calculation options below incorporate University of Minnesota nutrient recommendations.

Computer Software

The following software is available for developing manure management plans.  Aerial photos and/or maps of the land application sites must be attached prior to submitting the plan for review.

Nutrient Management Planner for Minnesota – Developed by the University of Minnesota Extension Service (UMES) and Natural Resources Conservation Service (NRCS). This Microsoft Access-based program will create a nutrient plan that can meet both NRCS and MPCA requirements.  A brochure, University of Minnesota – Manure Management Planner for Minnesota, provides a description and contact information.

MPCA Manure Management Planner – an Excel spreadsheet program, available on the MPCA Feedlots - Nutrient and Manure Management page.

Purdue University Software – A windows-based program for developing manure management plans. To find out more about the software or to download a copy of the Purdue Manure Management software program go to this website: www.agry.purdue.edu/mmp/.   Sensitive area management for Minnesota currently is not incorporated into this program.  When using this software, complete the form included in the document: “Manure Management Planning – Sensitive Areas Guidelines,  www.pca.state.mn.us/publications/wq-f6-38.pdf

Hand calculation methods

- “Manure Management Plan – a Step by Step guide for Minnesota feedlot operators” http://www.pca.state.mn.us/publications/wq-f8-09.pdf.

- NRCS forms "Field Nutrient Management Plan" CPA-23; "Farm Nutrient Management Plan" MN-CPA-38; and "Manure Nutrient Crediting" MN-CPA-35. Associated worksheets used to develop the plan include: MN-CPA-40, MN-CPA-41, MN-CPA-42, MN-CPA-43, and MN-CPA-46. These forms can be obtained by contacting your county NRCS office.

## Minnesota requirements for land application of manure

Each manure management plan and all manure applications in the state must, at a minimum, meet state requirements for rates, manure and soil testing, and setbacks from water and wells.  To find the requirements for land application of manure, see summary table 5-2 and the following:

- NPDES permitted feedlots – see General NPDES Permit pages 6 to 15
http://www.pca.state.mn.us/publications/feedlot-gp-permit-06.pdf

- Non-NPDES permitted feedlots – see 7020 rules pages 48 to 55
www.revisor.leg.state.mn.us/arule/7020/   or

- State requirements overview - Land Application of manure:  Minimum State Requirements www.pca.state.mn.us/publications/wq-f8-11.pdf

- Setbacks and high phosphorus soils – Applying Manure in Sensitive Areas: State Requirements and Setbacks. Fact sheet.

- Recipients of transferred manure – Manure Spreading Requirements Overview – for Recipients of Transferred Manure

- Stockpiling  Technical guidelines for stockpiling manure (March 2001)

- Which water types have state setback requirements?

www.pca.state.mn.us/publications/wq-f6-77.pdf

- Winter application at CAFOs – (Update available soon)

Table 5-2.  Overview of State Requirements for Land Application of Manure

| Requirements | Required for fewer than 100 AU? | Required for 100 to 299 AU? | Required for 300 to 999 AU? | Required for 1000 or more AU? |
| --- | --- | --- | --- | --- |
| Manage manure to prevent pollution of waters | Yes | Yes | Yes | Yes |
| Follow maximum nutrient rate limits | Yes | Yes | Yes | Yes |
| Maintain setbacks from sensitive features | Yes | Yes | Yes | Yes |
| Test manure for nitrogen and phosphorus content | No | Where stored manure is from over 100 AU | Where stored manure is from over 100 AU | Yes |
| Test soils for phosphorus | No | No | Yes | Yes |
| Develop and maintain a manure management plan | No | If permit is required | If permit is required, or if applied by non-certified person after Jan. 1, 2006 | Yes |
| Keep land application records | No | Yes | Yes | Yes |

Table 5-3. Minimum Setbacks for Manure Application in Sensitive Areas

|  | WINTER Frozen or snow-covered soils | NON-WINTER With immediate incorporation (within 24 hours) |  | NON-WINTER Not incorporated within 24 hours |  |
| --- | --- | --- | --- | --- | --- |
|  |  | With phos. mgmt. | No phos. mgmt. | With vegetated buffer | Without vegetated buffer |
| Lake, stream | 300’ | 25’ | 300’ | 100’ | 300’ |
| Intermittent stream,* DNR protected wetland,** drainage ditch without berms*** | 300’ | 25’ | 300’ | 50’ | 300’ |
| Open tile intake*** | 300’ | 0 | 0 | 300’ | 300’ |
| Well, mine, or quarry | 50’ | 50’ | 50’ | 50’ | 50’ |
| Sinkhole with no diversion berm | Downslope 50’ Upslope 300’ | 50’ | 50’ | Downslope 50’ Upslope 300’ | Downslope 50’ Upslope 300’ |

## County Requirements for Land Application of Manure

Where county ordinances require land application setbacks or practices that are more restrictive than state requirements, the local requirements must be followed.  Some county ordinances are available on the MPCA’s County Feedlot Ordinance Web page (available soon), and also the Minnesota Association of County Planning and Zoning Administrators Web page of county ordinance links.

## Keeping records of land application

After manure has been applied to cropland, records are required at feedlots with 100 or more animal units.  Optional forms for record keeping are listed below, along with instructions for the forms.  A spreadsheet is another tool that can be used to keep records of land application practices.

- Land Application Records for Feedlots over 300 AU – Microsoft Excel Worksheet  (4.3MB)

- Manure Application Record, 100 – 299 AU (form)

- Manure Application Record, 100 – 299 AU (instructions)

- Manure Application Record, 300+ AU (form)

- Manure Application Record, 300+ AU (instructions)

- Transfer of Manure Ownership, 300+ AU (form)

- Transfer of Manure Ownership, 300+ AU (instructions)

## 5.6    Reviewing Manure Management Plans

MMP review checklists

NPDES permits should not be issued until the MMP is reviewed and adequate.  MMP review prior to issuance of other permits is at the discretion of the CFO/MPCA. Use the checklist in the guidelines “Manure Management Plan Requirements and Checklist” available at: www.pca.state.mn.us/hot/feedlot-management.html .  Two electronic versions of this checklist (with spaces available for writing your comments) are available at

www.pca.state.mn.us/publications/wq-f8-14a.doc and

www.pca.state.mn.us/publications/wq-f8-14b.doc

Frequently asked questions

Answers to frequently asked questions about MMP review are located at www.pca.state.mn.us/publications/wq-f8-15.pdf

### Checking rates using spreadsheets

MPCA spreadsheet located at:  Feedlots - Nutrient and Manure Management

Extension Service Rate Calculator spreadsheet: http://www.manure.umn.edu/applied/marc/index.html

Checking to ensure adequate acreage

MPCA spreadsheet: Feedlots - Nutrient and Manure Management

Key parts of the plan to check

The MMP can be approved if the most important items are complete.  If minor items are missing or inadequate, we should notify the producer or person assisting the producer of the shortcomings, but not necessarily hold up permit issuance. The types of major problems listed below should be corrected before the MMP is approved, since these deficiencies may result in specific violations of land application requirements found in 7020.2225 subparts 2,3,6,7 or 8.

- Manure testing frequency is not described or does not show testing at least once every four years for non-CAFOs and annually for CAFOs;

- Estimated amount of manure generated is much lower in comparison with expected amounts, and past hauling records are not available to substantiate estimated amounts;

- Summer applications and no indication of cover crops;

- Winter application to fields with too many tile intakes to practically achieve the required 300 foot incorporation zone;

- Winter application at CAFOs without meeting all NPDES winter application restrictions;

- Field maps or aerial photos are not attached or the total field acreage is not available;

- Identified acreage is not sufficient to handle the nitrogen from manure (where ownership of manure is not transferred);

- Crop rotations are not described;

- Crop nitrogen needs are not described or deviate substantially from University of Minnesota recommendations;

- Manure is applied two years in a row to non-legumes (i.e. corn or small grains) and nitrogen credits from the previous year are not accounted for;

- Manure rates, combined with fertilizer additions, provide substantially more nitrogen than the crops need or remove.

- The sum of all manure to be applied to individual fields is far less than the expected manure generated at the feedlot;

- Setbacks to sensitive features such as waters, wells, sinkholes, etc. are not described or do not meet minimum requirements in rules;

- Plans or evidence of soil phosphorus testing is not evident;

- Manure is applied in special protection areas at a frequency that will build soil phosphorus levels on soils with already more than 21 ppm Bray (e.g. where manure applied over a six year period based upon crop phosphorus removal rate will potentially allow an increase in the soil phosphorus levels);

- Manure is to be applied to soils exceeding 
75 ppm Bray near tile intakes, lakes, streams, intermittent streams or wetlands over 10 acres; or exceeding 150 ppm Bray in other areas, and a phosphorus management strategy was not submitted or is inadequate.

Supplemental information

### Manure sampling and testing

- Soil testing

- Phosphorus Issues in Minnesota

- Calculating Manure Application Rates

- Setback Requirements

### Calibrating manure spreaders

- Record Keeping (Available soon)

### Benefits of manure – runoff reductions with incorporated manure – MPCA Literature Review

- Minnesota NRCS – Manure Nutrient Management

- Commercial Animal Waste Technician Licensing