| Stormwater Checklist Complete checklist prior to determination of allocations Policy and guidance referred to below can be found at http://www.pca.state.mn.us/water/stormwater/impairedwaters.html |  |  |
| --- | --- | --- |
| Individual Non-MS4 Stormwater |  |  |
| Are there individual non-MS4 NPDES stormwater permits within the study area (e.g. Wal-Mart, etc.)? |  |  |
| No | Go to Construction Stormwater |  |
| Unsure | Contact a Stormwater-TMDL liaison |  |
| Yes | The TMDL must have a WLA for the permit(s). Contact a Stormwater-TMDL liaison. |  |
| Go to Construction Stormwater |  |  |
| Construction Stormwater Is the impairment for turbidity, dissolved oxygen, nutrient/eutrophication biological indicators, aquatic macroinvertebrate bioassessments, aquatic plant bioassessments, or fish bioassessments? |  |  |
| No | TMDL does not require a WLA for construction stormwater. |  |
| Go to Industrial Stormwater |  |  |
| Yes | The TMDL must have a WLA for construction stormwater. For more information see MPCA Policy for Setting Wasteload Allocations for Regulated Stormwater or contact a Stormwater-TMDL liaison. |  |
| Are there specific BMPs beyond those described in the Construction Stormwater General Permit that should be implemented? |  |  |
| Yes | Contact a Stormwater-TMDL liaison. |  |
| Unsure | Contact a Stormwater-TMDL liaison. |  |
| No | # TMDL must contain standard language about construction permits (see MPCA Policy for Setting Wasteload Allocations for Regulated Stormwater. |  |
| Go to Industrial Stormwater |  |  |
| Industrial Stormwater |  |  |
| - Is the pollutant of impairment a benchmark pollutant for one or more industries? (for list of benchmark pollutants, see http://www.pca.state.mn.us/water/stormwater/impairedwaters.html) |  |  |
| No | TMDL does not require a WLA for industrial stormwater. |  |
| Yes | The TMDL must have a WLA for industrial stormwater. Include permit numbers for each industrial facility if they are readily available. For more information see MPCA Policy for Setting Wasteload Allocations for Regulated Stormwater or contact a Stormwater-TMDL liaison. |  |
| - Are there specific BMPs, monitoring, or compliance schedules beyond those described in the Industrial Stormwater Multisector General Permit that should be implemented? |  |  |
| Yes | Contact a Stormwater-TMDL liaison. |  |
| Unsure | Contact a Stormwater-TMDL liaison. |  |
| No | # If a WLA has been developed for industrial stormwater, the TMDL must contain standard language about industrial permits (see MPCA Policy for Setting Wasteload Allocations for Regulated Stormwater. |  |
| Go to Municipal Stormwater |  |  |
| Municipal Stormwater |  |  |
| 1. Are there regulated MS4s within the study area? |  |  |
| Unsure | Contact a Stormwater-TMDL liaison. |  |
| No | Do you anticipate regulated MS4s will exist within the study area in the future? |  |
|  | No | TMDL does not require a WLA for MS4 stormwater. |
|  | Unsure | Contact a Stormwater-TMDL liaison |
|  | Yes | Consider setting WLAs to account for future MS4s. See Guidance on What Discharges Should be Included in the TMDL Wasteload Allocation for MS4 Stormwater |
|  | End checklist |  |
| Yes | The TMDL must have a WLA for municipal stormwater and must contain permit numbers for each MS4. For more information contact a Stormwater-TMDL liaison or see MPCA Policy for Setting Wasteload Allocations for Regulated Stormwater. |  |
|  | Are there specific BMPs that should be implemented? |  |
|  | Yes | Contact Stormwater-TMDL liaison and go to Question 2. |
|  | No | Go to Question 2. |
| 2. Is future growth or future load transfer an issue for this TMDL? |  |  |
|  | Yes | Apply guidance (See Guidance on What Discharges Should be Included in the TMDL Wasteload Allocation for MS4 Stormwater) or contact a Stormwater-TMDL liaison. |
|  | No | Estimate WLA based on current land use. |
|  | Unsure | Contact a Stormwater-TMDL liaison. |
| End checklist |  |  |

Recommended points in a TMDL process for contacting Stormwater-TMDL liaisons:

| Item | TMDL has or is likely to have a regulated MS4 WLA | TMDL does not or is not likely to have a regulated MS4 WLA | Recommended review time for liaison |
| --- | --- | --- | --- |
| Stakeholder process developed | x |  |  |
| Work plan submitted | x |  | 1 week |
| WLAs calculated | x | x |  |
| Draft TMDL | x | x | 2 to 3 weeks |
| TMDL public noticed | x | x |  |
| Implementation Plan started | x |  |  |
| Draft implementation plan | x |  | 2 to 3 weeks |

Stormwater-TMDL Liaisons

Anna Kerr: anna.kerr@pca.state.mn.us; Mike Trojan: mike.trojan@pca.state.mn.us