|  | SOP 2.04 Personal Protective Equipment Information for HHW Programs |
| --- | --- |

Note: This guidance document is for informational purposes only and outlines basic employer requirements. The MPCA does not guarantee that this document satisfies all applicable compliance requirements. It is the sole responsibility of county-specific Household Hazardous Waste (HHW) Program employer to determine if OSHA, DOT and RCRA standards have been met.

# 1.	Introduction

This Program shall ensure staff is provided with the proper selection, procurement, and use of personal protective equipment (PPE) and/or clothing. Engineering and administrative controls are the preferred methods for controlling hazards. PPE is used to protect Facility staff from exposure or injuries.

# 2.	PPE Program

2.1  Needs Assessment and Certification of PPE

The designated PPE Coordinator for this Program is [the HHW Program Manager] and shall conduct a comprehensive needs assessment to identify and establish required PPE. PPE requirements shall be tailored to the unique conditions and operations of this facility and updated as job duties or hazards change. The designated PPE Coordinator shall model compliance by wearing required PPE and adhering to the program policies, hazard assessment and PPE selection guidelines.

2.2 Contractor Compliance

HHW volunteers or contractors providing services under this Program must also comply with the PPE requirements outlined in this SOP. The designated PPE Coordinator shall also ensure compliance with this Program’s AWAIR Program.

2.3 Provision of PPE

All required PPE shall be provided by the Program at no cost to Facility staff. A PPE reference chart identifying specific tasks and corresponding required PPE shall be listed and posted for staff reference; see Attachment A.

2.4 Determination of PPE Levels

Good housekeeping will minimize chemical and biological hazards; the selection and level of PPE required for each specific task shall be determined based on existing environmental conditions and the professional judgment of experienced Facility staff. Ensure a written certification confirming that a workplace Hazard Assessment has been completed and is routinely updated. The PPE hazard assessment shall include the following elements:

- A title identifying it as the certification of hazard assessment

- Documentation of the workplace evaluation

- The name and signature of the individual certifying completion of the assessment

- The date(s) the hazard assessment was performed

# 3.	PPE Training Requirements

3.1 Implementing a Training Program

The designated PPE Coordinator is responsible for administering, implementing, and enforcing the PPE policy. All Facility staff shall receive training on proper PPE use, including:

- Selection and appropriate use

- Donning, doffing, adjustment, and wearing procedures

- Limitations and service life

3.2 PPE Training Protocol

This Program shall ensure that all HHW staff required to use PPE receive appropriate training. The designated PPE Coordinator is responsible for coordinating and certifying all PPE training. Training shall:

- Be conducted initially, prior to PPE use

- Include waste evaluation prior to handling to ensure appropriate PPE use

- Be repeated when noncompliance is observed

- Encourage job rotation to reduce repetitive motion injuries

- Be updated when workplace conditions change and render previous training obsolete

- Encourage staff to report and seek treatment for work-related health concerns

- Continue until staff demonstrate full understanding and competency

# 4. 	PPE Selection

PPE shall be provided, used, and properly maintained wherever workplace hazards exist such as processing, environmental, chemical, or mechanical risks. Chemical exposure may cause injury or staff exposure through skin absorption, inhalation, or physical contact.

4.1 Eye and Face Protection

Eye and face protection must be worn whenever exposure to potentially injurious hazards exists (e.g., flying particles, liquid chemicals, corrosives, gases, vapors, or UV radiation). All protective eyewear shall:

- Provide eye and side protection (detachable or permanent)

- Splash guards to prevent exposure to chemicals

- Fit comfortably and securely

- Be worn over prescription lenses when necessary

4.2 	Foot Protection

In accordance with OSHA requirements, which may include steel toed boots; sandals and open-toe shoes are prohibited. Protective footwear shall be worn when risks of foot injury exist, including exposure to:

- Compression, impact, or rolling objects

- Electrical hazards

- Slipping or chemical exposure

- Temperature extremes

- Puncture hazards from sharp objects

4.3 Hand and Body Protection

Appropriate hand and body PPE shall be selected using glove manufacturer specifications and worn when hazards include:

- Cuts and punctures: from tools, machinery, or sharp objects

- Thermal exposure: from extreme cold or hot work

- Bloodborne pathogens: during first aid or biohazard cleanup

- Chemical exposure: through direct skin contact

- Proper ergonomic work practices: to prevent chemical exposures or repetitive motion (tasks such as opening paint cans)

- Machine guards: to prevent cuts, punctures, and abrasions

- Entanglement: from loose gloves, clothing, or jewelry near moving equipment

4.4 Head protection

Head protection (e.g., hard hat) shall be worn if potential for injury from falling objects exist. All head protection shall:

- be used according to manufacturer’s instructions

- not be altered in any way

- include bump caps as an appropriate alternative (if exposure to scalp injuries exist)

4.5 Hearing protection

Facility staff shall wear hearing protection when exposed to noise levels in excess of 85 dB(A) for an eight-hour time-weighted average.

4.6 Respiratory protection

Site specific ventilation or exhaust systems are effective engineering control tools. Review Program SOP specific for procedures on both mandatory and voluntary use of respirators. Engineering and work-practice controls shall be implemented to maximize air quality protection. Staff shall be pre-tested and authorized to use full face respirators or face masks.

# 5. 	Documentation and record retention

The designated PPE Coordinator shall maintain and securely file all records related to personal protective equipment (PPE) training. Documentation shall be reviewed and updated as facility operations or procedures change. Training documentation must include, at a minimum:

- Staff training records (e.g., course title, participant names, training dates, instructor(s), and session agendas/outlines)

- The PPE Equipment and Clothing Compliance Checklist: see Attachment B.

- Staff written approval for respirator use.

- Routine documentation of respirator care and maintenance.

### Attachment A

### Personal Protective Equipment Requirements for:___________________HHW Program.

___________________ is the Designated Person responsible for PPE Program Implementation.

Please complete and post

| PPE | Receiving Wastes | Sorting Wastes |  | Bulking Flammables, Solvents | Bulking Latex Paint | Lab-Packing | Spill Cleanup (incidental) | Traffic Control | ID of Unknowns |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chemical-resistant gloves | R |  | R | R | O | R | R | O | R |
| Inner protective gloves | O |  | O | R | O | R | R | O | R |
| Tyvek protective suit | O |  | O | O | O | O | O | O | O |
| Apron or coverall | R |  | R | R | R | R | R | O | R |
| Chemical-resistant protective suit | O |  | O | R | O | O | R | O | O |
| Safety glasses with side shields | R |  | R | R | R | R | R | O | R |
| Goggles | O |  | O | R | O | O | R | O | O |
| Face shields | O |  | O | O | O | O | O | O | O |
| Reinforced footwear or toe caps | R |  | R | R | R | R | R | R | R |
| Chemical-resistant foot covers | O |  | O | O | O | O | O | O | O |
| Non-chemical-resistant foot covers | O |  | O | O | O | O | O | O | O |
| Respiratory protection | O |  | O | R | O | R | R | O | R |
| Safety headgear | O |  | O | O | O | O | O | O | O |
| Hearing protection | O |  | O | O | O | O | O | O | O |
| Long pants | R |  | R | R | R | R | R | R | R |
| Long sleeves | R |  | R | R | R | R | R | R | R |
| Reflective traffic vest | O |  | O | O | O | O | O | R | O |

These recommendations are intended to provide guidance. Adjust list as needed to make specific to this Program PPE policy. An “R” indicates PPE usage is required. An “O” indicates PPE usage is optional.
Attachment B

## Personal Protective Equipment and Clothing Compliance Checklist

|  | Yes No N/A |
| --- | --- |
| 1. Are protective goggles or face shields provided and worn where there is any danger of flying particles or corrosive materials? |  |  |
| 2. Are approved safety glasses required to be worn at all times in areas where there is a risk of eye injuries exist? |  |  |
| 3. Are Facility staff who need corrective lenses (glasses or contacts) and work in environments with potential harmful exposures, being provided with approved safety glasses, protective goggles, or other medically approved precautionary devices? |  |  |
| 4. Is proper PPE provided where Facility staff could be injured, or where there is reasonably anticipated exposure to corrosive liquids, chemicals, blood, or other potentially infectious materials? |  |  |
| 5. Is appropriate foot protection required where there is the risk of foot injuries? |  |  |
| 6. Is all PPE maintained in a sanitary condition and ready for use? |  |  |
| 7. Are eye wash stations and drench showers located within 10 seconds of where Facility staff are exposed to injurious corrosive materials? |  |  |
| 8. Are food or beverages consumed in areas where there is no exposure to toxic or potentially infectious materials? |  |  |
| 9. Is hearing protection provided when sound levels exceed those of the OSHA noise standard? |  |  |
| 10. Are adequate work procedures and PPE provided and used when cleaning up hazardous materials or spills? |  |  |
| 11. Are there appropriate procedures in place for disposal of contaminated PPE? |  |  |

______________________________________  ___________________________________________
Audited/Verified  by (print)	Date		Staff signature				Date