EXAMPLE: TEST PLAN FOR MISCELLANEOUS SOURCES

Proposed test date(s):

(Test plan postmark or e-mail date submittal date triggers 30 day notification period)

PART I.  GENERAL INFORMATION

1.	Name and street address of emission facility:

If company cannot receive mail at the street address, a mailing address should also be indicated.

Name, Title, Telephone and Facsimile number and e-mail address of contact person at emission facility:

This one person will receive correspondence & participate in the pretest meeting.  They should be knowledgeable about the emission unit being tested, and be able to relay operating and documentation requirements to appropriate company staff.

2.	Permit File Number:

AQ File

3.	Reason the emission unit(s) is (are) to be tested:

(Performance test to demonstrate compliance with:  Permit No, Stipulation Agreement dated xx/xx/xx, Administrative Penalty Order dated xx/xx/xx, Notice of Noncompliance Dated xx/xx/xx, Notice of Indeterminate Compliance dated xx/xx/xx, or refer to the Test Plan cover sheets for other types of test classifications.  Please be as descriptive as possible to ensure your test plan is reviewed accurately.)  Note that, in all cases, retests following noncompliance or indeterminate compliance tests should be indicated as Retest # (1, 2, etc.) where applicable.

4.	Emission unit(s) and associated control equipment layout schematic and sampling port location(s) of emission unit(s) to be tested:

Provide a schematic giving a simple block diagram of unit(s) being tested, plus their associated air pollution control equipment. .  This can be combined with the port location information, as appropriate.

Drawings showing location of the test ports must be submitted for approval as part of  the test plan.  Drawings must depict a diagram of the flue gas system from the outlet of the emission unit, control equipment to the stack vent exit. All measurements should be verified.  The following dimensions should be included:

·inside stack diameter at the location of the sampling ports

·distance between the sampling ports for non-circular stacks

·angle between ports for circular stacks

·upstream and downstream distance from the sampling ports and/or any flow disturbance (e.g. elbows, angles, size and shape transitions, straightening vanes, etc.)

5.	Name of Independent Testing Company, contact person, e-mail address, telephone and facsimile number:

PART II. TESTING REQUIREMENTS

1.	The following is a description of the Pollutant(s) to be tested, and the applicable emission limit(s), and the applicable rule(s) or regulation(s) for each emission limit:

| Stack Vent No.: Emission Unit No. | Limitation Basis of Pollutant Tested | Pollutant Tested and Applicable Emission Limit | Specific Methods/Procedures Required Citation: |
| --- | --- | --- | --- |
| Stack Vent #01: Dryer #01: (Dryer #02 not operating during testing) | Minn. R. 7011.0715, Subp. 1.(A) | Particulate Matter: Variable, but never to exceed X gr/dscf or X lb/hr. And, never to exceed 0.3 gr/dscf | Minn. R.7011.0725 (Document, regulation, Rule, or Statute reference) |

Operating Data to be recorded during the Test:

1.	Operating conditions of each source tested and its associated pollution control equipment will be documented in the test report.   Documentation of operating conditions includes all parameters listed in Part III.

No test report will be accepted without complete documentation of process conditions during the test.

2.	The following is a detailed description of the procedure for fuel sampling and analysis to be followed for the applicable emission unit.

Read any applicable NSPS regulation, the permit and Exhibit D carefully to ensure all fuel specific analysis parameters are addressed.  These are part of  the compliance demonstration requirements.  Describe the sample collection method, time interval, and the analysis parameters.  For example, One coal sample shall be taken every fifteen minutes from each feeder of each fuel burned during the test.  The sample will be taken as close as possible to the burner, (somewhere in feeding line) to be representative of the fuel burned at the time of the test. No fuel analysis for natural gas is required unless specified by an applicable NSPS or other regulation.

- Gross Heating Value	Analysis Method

- Sulfur, % by weight	Analysis Method

- Ash, % by weight	Analysis Method

- Moisture Content, % by weight	Analysis Method

- Others as needed, or as specified by Exhibit D for fuel types      	Analysis Method

PART III.  OPERATING CONDITIONS

(Read the italicized section below the table for examples of parameters that need to be considered for process/operating conditions.  Refer to the Test Plan cover sheets for examples of parameters to be monitored for air pollution control equipment.)

1.	The following contains a detailed description of  the emission unit(s) to be tested.  Detailed descriptions of record keeping that include the specific time interval, and other parameters listed that will determine production, operating capacity, and/or operating conditions during testing are also included:

Stack Vent No. 01:  Dryer No. 01

Process Equipment Description for units to be tested: Type of equipment, mfr. & model, rated capacity, fuel type

Process Equipment Parameter Monitoring During Performance Test: Process Rates/Operating Conditions During Testing: Read items A & B below before filling in this section. Include specific parameter and time interval for documentation (Refer to the Test Plan cover sheets for examples). Be sure to indicate relevant information for each item. These are important considerations for worst case conditions. If any are applicable, explain what the proposed operating conditions and documentation that pertain to these are. Think about the unit tested and what the company is capable of monitoring. Always list the unit of measure as well as the time interval the company will present process/operating rates at. These must be summarized for each run. A. Specify operating conditions at the time of the test for each source being tested. Specify if the source has to be tested under more than one combination of operating conditions. Consider the following for monitoring: 1. any physical parameters 3. any process rates-raw material input, 2. any chemical parameters or product output B. If the emission unit being tested is a batch type process, it should be indicated as such. The company should specify at what time, within the batch, the test should be conducted as a function of the emission rate profile. Consider the following for monitoring: 1. process rates 4. physical parameters 2. raw material input or product output 5. chemical parameters 3. fuels 6. duration of batch process

Control Equipment Description: Type of equipment, mfr. & model, operating ranges required by permit or mfr. recommended

Control Equipment Operating Parameter During Test: Include specific parameter and time interval for documentation (Refer to the Test Plan cover sheets for examples) Attached is a schematic of the ESP servicing the unit to be tested. The orientation of the ESP sections are clearly labeled to correspond to the identification that shall be used when documenting operating parameters listed above. The inlet and outlet are identified.

Include any calculations, process, control equipment parameters that are applicable.

2.	For each process unit listed in Part III, Item 1, the normal range of process or operating rates for each emission unit are listed below.  The proposed test conditions listed in Part III, Item 1, are considered worst case in accordance with Minn. R. 7017.2025, subpart 2 as indicated below:

Stack Vent No. 01:  Dryer No. 01

| Normal Range of Process or Operating Rates | Rationale For Worst Case |
| --- | --- |
| Describe | Maximum achievable process/operating rate |

3.	If emission unit being tested shares a stack, indicate the operating conditions of other emission units during testing.  If no stack is shared delete this item.

PART IV.  TEST METHODS

1.	The following is a description of the methods, number of test runs, length of test runs, and sampling rate of each pollutant:

This listing does not include all acceptable methods, or all pollutants the Company may be required to test for, it is merely an example.  Specific information for this section should be obtained from the permit, applicable regulations, and the testing company.  Refer to page 1 of the Test Plan cover sheets.

A.	EPA Method 1 for the location of sampling ports and points.

Absence or presence of cyclonic flow will be verified for any stack/vent/duct configuration that may not have parallel stack gas flow due to cyclones, inertial demisters following venturi scrubbers, tangential inlets, or other duct configurations which tend to induce swirling.  Correction of cyclonic flow by straightening vanes will be verified.  If the location does not meet the minimum requirements in EPA Method 1, the testing firm will conduct the flow pattern evaluation and testing according to alternative procedures in  part 2.5 of EPA Method 1.  Documentation will be provided.

B.	EPA Method 2 for velocity and volumetric flow rate.  One measurement concurrently with each test run for every pollutant.

C.	EPA Method 3 for gas analysis. One test run on an integrated sample taken concurrently with each test run for every pollutant.

D.	EPA Method 4 for the determination of moisture in the flue gases.  One test run concurrently with each test run for every pollutant.

E. EPA Method 5 as amended by Minn. R. 7011.0725 for the concentration of particulate matter including organic condensables.  Results are to be reported both as filterable particulate matter containing organic condensable (where the limit includes organic condensables), and as filterable particulate matter excluding organic condensables (where the limit does not include organic condensables). Run time:  60 minutes, Sample volume:  32 dscf (0.90 dscm)

F.	EPA Method 201A and 202 for the concentration of PM10.  The sampling time for each run shall be at least 60 minutes and the minimum sampling volume will be 32 dscf (0.9 dscm).  Results for PM10 include condensable particulate matter emissions as determined by Method 202.

F.	EPA Method 9 as amended by Minn. R. 7017.2060 for visual determination of opacity. One hour of observations, concurrently with a test run for particulates.  If this is an initial compliance test subject to NSPS regulations, then three one-hour runs of opacity are required concurrently with particulate runs.

2.	If any alternative or equivalent methods are proposed, include a summary of the reasons for the proposal.  If it is not applicable delete items 2 and 3.

3.	For a non-reference test method, include a statement of the detection limit and the degree of accuracy of that method at the expected emission rate and under the conditions of the performance test.

Note:	Sampling times and rates will be increased if necessary to ensure that the detection limit for each pollutant is below the applicable emission limit, using the equation in Minn. R. 7017.2045, Subp. 6.

PART V. Continuous Emissions Monitoring (CEM)

Note that if CEM data is to be used for demonstrating compliance with applicable emission limits, the CEM equipment must be certified and meet all applicable requirements and specifications.  Be sure the permit or applicable regulation allows for use of the CEM for compliance purposes.

PART VI.  OTHER

1. 	Pollutant(s) to be tested will be reported in terms of the applicable emission limit units.

2.	Testing schedules and testing firm.

If testing will consist of several sources, please format them as follows:

| 08/31/95 - Thursday | 10/01/95 - Friday | 10/05/95 - Tuesday |
| --- | --- | --- |
| Set - Up | PM & VE on Dryer #01: 0800-1400 | XX on Dryer #XX: xxxx-xxxx |

3.	Description and date of last maintenance work done before the test:

No major rehabilitation or cleaning before the test other than normal maintenance operation done on a routine basis will be conducted before testing.  A description of any maintenance work done before the test and the normal schedule followed will be included as part of the report. Indicate what the standard maintenance schedule is.

4.	One complete test report (one hard copy) shall be submitted within 45 days after the date of the test.  A copy of the microfiche or CD report shall be submitted within 105 days after the date of the test.

If a different time frame is specified in the permit or other compliance document, change the submittal dates accordingly.

Test Plans, Hard Copy Reports, and Microfiche or CD Copy submittals will be addressed to: Compliance Tracking Coordinator, Compliance and Enforcement Section, Industrial Division, Minnesota Pollution Control Agency, 520 Lafayette Road, St. Paul, Minnesota 55155-4194