|  | UST shear valve inspection form Underground Storage Tanks (UST) Program Doc Type: Compliance Certification |
| --- | --- |

Purpose:  This form is for documenting the inspection of the shear valves. See PEI/RP1200, Section 10 for inspection procedures.

## Facility information

| Facility name: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Facility address: |  |  |  |  |  |  |  |  |  |  | Facility ID#: |  |  |
| City: |  |  |  |  |  |  |  | State: |  |  |  | Zip code: |  |
| Owner name: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mailing address: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| City: |  |  |  |  |  |  |  | State: |  |  |  | Zip code: |  |
| Phone: |  |  |  |  |  | Fax: |  |  | Email: |  |  |  |  |

Testing information

| 1. Product grade |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. Dispenser ID# |  |  |  |  |  |  |  |  |  |  |  |
| 3. Shear valve brand and model |  |  |  |  |  |  |  |  |  |  |  |
| 4. Is the shear valve a double poppet? |  | Yes No | Yes No |  |  | Yes No |  | Yes No | Yes No | Yes No |  |
| 5. Is the shear valve secondarily contained? |  | Yes No | Yes No |  |  | Yes No |  | Yes No | Yes No | Yes No |  |
| 6. Is the shear valve rigidly anchored to the dispenser box frame or dispenser island per the manufacturer’s specifications? |  | Yes No | Yes No |  |  | Yes No |  | Yes No | Yes No | Yes No |  |
| 7. Is the shear section positioned between ½ inch above or below the top surface of the dispenser island? |  | Yes No | Yes No |  |  | Yes No |  | Yes No | Yes No | Yes No |  |
| 8. Is the lever arm free to move? |  | Yes No | Yes No |  |  | Yes No |  | Yes No | Yes No | Yes No |  |
| 9. Does the lever arm snap shut the poppet valve? |  | Yes No | Yes No |  |  | Yes No |  | Yes No | Yes No | Yes No |  |
| 10. Can any product be dispensed when the product shear valve is closed? |  | Yes No | Yes No |  |  | Yes No |  | Yes No | Yes No | Yes No |  |
| A “No” in lines 6-9 or a “Yes” for line 10 indicates a failure. |  |  |  |  |  |  |  |  |  |  |  |
| Test results: |  | Pass Fail | Pass Fail |  |  | Pass Fail |  | Pass Fail | Pass Fail | Pass Fail |  |
| Comments: |  |  |  |  |  |  |  |  |  |  |
| Testing company name: |  |  |  | Tester’s name: |  |  |  |  |  |  |
| Date (mm/dd/yyyy): |  |  |  |  | Tester’s signature: |  |  |  |  |  |