|  | Emergency Response Plan NPDES and SDS Permit Program Feedlot Program Doc Type: Permit Application |
| --- | --- |

## Applicability:  This Emergency Response Plan is to be used in case of an emergency spill, leak, or failure at the production facility or land application area and to assist with response to catastrophic animal mortality events (barn fires, tornadoes, etc.). You must submit this form as part of an application for National Pollutant Discharge Elimination System (NPDES) or State Disposal System (SDS) feedlot permit coverage.

| ## Facility name: |  |  | ## Feedlot registration no.: |  |  |
| --- | --- | --- | --- | --- | --- |
| ## Owner/Operator name: |  |  | ## Feedlot permit no.: |  |  |

List of critical phone numbers and contacts

|  | Contact person (or Company) | Phone number |  |
| --- | --- | --- | --- |
| Emergency contacts |  |  |  |
| - Fire/Ambulance | ------------------------------------------ | 911 |  |
| - County Sheriff |  |  |  |
| Agency contacts |  |  |  |
| - Minnesota Duty Officer | ------------------------------------------ | 1-800-422-0798 | Provide the Minnesota Duty Officer: - Your contact information - Incident location, date, and time - For spills - spill type - spill amount - surface water or field tile impacted - Progress made in response to the spill or catastrophic mortality event |
| - Minnesota Pollution Control Agency (MPCA) Field Office |  |  |  |
| - County Feedlot Officer (CFO) |  |  |  |
| - Board of Animal Health Contact |  |  |  |
| Other contacts |  |  |  |
| - Insurance company |  |  |  |
| - Gopher State One Call | ------------------------------------------ | 1-800-252-1166 |  |
| Local vendors for spill and/or catastrophic mortality response assistance |  |  |  |
| - Manure pumper |  |  |  |
| - Manure loading equipment |  |  |  |
| - Earth moving equipment |  |  |  |
| - Tiling equipment |  |  |  |
| - Containment/Absorption materials (hay, straw, cornstalks, sawdust) |  |  |  |

| Manure Spill Emergency Response Procedures* |  |
| --- | --- |
| 1. Immediately stop the source of a liquid manure leak or spill: - Turn off pumps or valves - Clamp hoses or park tractor on hoses 2. Contain spilled manure: - Use skid loader or tractor with blade to make berms - Install bale checks and block downstream culverts - Insert sleeves around tile intakes (or plug/cap) - Use tillage equipment to work ground ahead of spill - Use absorptive materials 3. Make necessary phone calls as listed in the chart above: - Notify Minnesota Duty Officer at 1-800-422-0798 - Notify sheriff’s office if spilled on public roads or right-of-ways | - 4. Cleanup: - Clean up spill immediately from road and roadside - Clean up all material, including the contaminated soil, as soon as possible by scraping, or by other means - Land apply manure at agronomic rates or place manure back in the manure storage area/ solid manure stockpile - Follow recommendations of MPCA staff and/or CFO - Restore site to its original conditions - If rain is expected prior to completion of cleanup; actions need to be taken to contain manure contaminated runoff from solid manure spills - 5. Document your actions: - Keep records of all actions related to the spill and follow up activities |
| - *A detailed site map should be displayed on site to assist employees identify sensitive receptors near the facility (surface water, wells, tile intakes, etc.). |  |

| Catastrophic Animal Mortality Response |  |  |  |
| --- | --- | --- | --- |
| 1. Make necessary phone calls as listed in the chart above: - Notify Minnesota Duty Officer at 1-800-422-0798 - Notify Minnesota Board of Animal Health - Notify MPCA and CFO - 2. Cleanup - Dispose of mortalities according to recommendations of MN Board of Animal Health Representative - Locate disposal area for mortalities to prevent impacts to surface and/or groundwater (consult MPCA/CFO) - 3. Document your actions - Keep records of all actions related to the animal mortality disposal activities | - If burial of animal mortalities is necessary, the burial site must meet the following: - Located 1000 feet from lakes and 300 feet from rivers and streams - Mortalities are not buried within 5 feet of the seasonal water table - Mortalities are not buried within 10 feet of karst susceptible bedrock - Soils are not sandy or gravelly - Describe approximate location(s) of potential burial site(s) below: |  |  |